UTILITIES

THE LOCATIONS OF THE UNDERGROUND UTILITIES SHOWN ON THE PLANS

ARE AS OBTAINED FROM THE OWNERS OF THE UTILITIES AS REQUIRED BY SECTION 153.64 OF THE OHIO REVISED CODE. ODOT ASSUMES NO RESPONSIBILITY FOR THE LOCATION OR THE DEPTHS OF THE UNDERGROUND FACILITIES SHOWN ON THESE PLANS. AT LEAST 48 HOURS BEFORE DIGGING, THE CONTRACTOR SHALL CALL THE OHIO UTILITIES PROTECTION SERVICE

AT THE NUMBER LISTED ON THE TITLE SHEET. NON-MEMBER UTILITY COMPANIES MUST BE CALLED DIRECTLY.

LISTED BELOW ARE ALL UTILITIES LOCATED WITHIN THE PROJECT CONSTRUCTION LIMITS TOGETHER WITH THEIR RESPECTIVE

OWNERS:

POWER

AMERICAN ELECTRIC POWER CO. (DISTRIBUTION)

38831 S.R. 7

REEDSVILLE, OHIO 45772

ATTN: CLARKE SAUNDERS

740-985-3054

CMSAUNDERS@AEP.COM

TELECOM

BRIGHTSPEED (FORMERLY LUMEN)

2025 AKRON RD.

WOOSTER, OHIO 44691

ATTN: JEFF SCHOONOVER

980-376-1554

330-464-9020

JEFFERY.L.SCHOONOVER@BRIGHTSPEED.COM

RELOCATIONS@BRIGHTSPEED.COM

SPECTRUM CABLE TV

3770 EAST LIVINGSTON AVE.

COLUMBUS, OHIO 43227-2280

ATTN: ANTHONY ADAMS

614-827-7971

ANTHONY.ADAMS@CHARTER.COM

GAS

COLUMBIA GAS OF OHIO

3550 JOHNNY APPLESEED CT.

COLUMBUS, OHIO 43231

ATTN: TARA NEMCIK

614-813-1402

TNEMCIK@NISOURCE.COM

OHIO CUMBERLAND GAS COMPANY

20718 DANVILLE-AMITY ROAD

MOUNT VERNON, OHIO 43050

ATTN: MARK RAMSER

740-392-2941

MARKRAMSER@HOTMAIL.COM

UTILITIES (CONT’D)

NATIONAL GAS AND OIL COOPERATIVE

120 O’NEIL DRIVE

HEBRON, OHIO 43025

ATTN: WILL POLING

740-641-8751

WPOLING@THEENERGYCOOP.COM

ROUNDING

THE ROUNDING AT SLOPE BREAKPOINTS SHOWN ON THE TYPICAL SECTIONS APPLIES TO ALL CROSS SECTIONS, EVEN THOUGH OTHERWISE SHOWN.

# CONTINGENCY QUANTITIES

THE CONTRACTOR SHALL NOT ORDER MATERIALS OR PERFORM WORK FOR

ITEMS DESIGNATED BY PLAN NOTE TO BE USED "AS DIRECTED BY THE ENGINEER" UNLESS AUTHORIZED BY THE ENGINEER.  THE ACTUAL WORK LOCATIONS AND QUANTITIES USED FOR SUCH ITEMS SHALL BE INCORPORATED INTO THE FINAL CHANGE ORDER GOVERNING THE COMPLETION OF THIS PROJECT.

# ITEM 202, PAVEMENT REMOVED, AS PER PLAN

THIS ITEM INCLUDES REMOVAL OF EXISITNG ASPHALT CONCRETE PAVEMENT ALONG WITH ASPHALT CURB AT THE LIMITS SHOWN IN THE PLANS.

# ITEM 609, CURB, TYPE 4-A, AS PER PLAN

THIS ITEM INCLUDES ONE (1) FOOT OF GUTTER, POURED INTEGRAL TO CURB,

TO FILL THE VOID FROM ITEM 202, PAVEMENT REMOVED AS SHOWN IN TYPICAL SECTIONS. THE CROSS SLOPE OF THE GUTTER SHOULD MATCH THE EXISTING TRAVELED LANE.

# ITEM 630, GROUND MOUNTED SUPPORT, NO 3 POST, AS PER PLAN

TYPE S SQUARE POSTS ALONG WITH POST ANCHOR BASE INSTALLATION PER

TC-41.20 SHALL BE USED FOR ALL GROUND-MOUNTED SUPPORTS.

ITEM 653, TOPSOIL FURNISHED AND PLACED, AS PER PLAN

# ON-SITE TOPSOIL MEETING THE REQUIREMENTS OF CMS 659.05 IS

# ALLOWED FOR USE ON THIS PROJECT, AS DIRECTED BY THE ENGINEER.

TOPSOIL SHALL BE USED THROUGHOUT PROJECT LIMITS, UNLESS

DIRECTED OTHERWISE BY THE ENGINEER.

SEEDING AND MULCHING

THE FOLLOWING QUANTITIES ARE PROVIDED TO PROMOTE GROWTH

AND CARE OF PERMANENT SEEDED AREAS:

ITEM 653, TOPSOIL FURNISHED AND PLACED, AS PER PLAN:  SEE SHEET 9

ITEM 659, SOIL ANALYSIS TEST:  2 EACH

ITEM 659, SEEDING AND MULCHING, CLASS 1:  SEE SHEET 9

ITEM 659, REPAIR SEEDING AND MULCHING:  27 SY

ITEM 659, INTER-SEEDING:  27 SY

ITEM 659, COMMERCIAL FERTILIZER:  0.07 TON

ITEM 659, LIME:  0.11 ACRE

ITEM 659, WATER:  3 MGAL

SEEDING AND MULCHING SHALL BE APPLIED TO ALL AREAS OF

EXPOSED SOIL BETWEEN THE RIGHT-OF-WAY LINES, AND WITHIN

THE CONSTRUCTION LIMITS FOR AREAS OUTSIDE THE RIGHT-OF-WAY

LINES COVERED BY WORK AGREEMENT OR SLOPE EASEMENT.

QUANTITY CALCULATIONS FOR SEEDING AND MULCHING ARE

BASED ON THESE LIMITS.

UNRECORDED DRAINAGE

FURNISH NO CONTINUANCE FOR ANY UNRECORDED UNTREATED

NON-STORM WATER DRAINAGE SUCH AS UNTREATED SEPTIC, UNTREATED WASTEWATER, UNTREATED CURTAIN/GRADIENT DRAINS, AND UNTREATED FOUNDATION FLOOR DRAINS DISTURBEDBY THE WORK.  PLUG ANY UNRECORDED UNTREATED NON-STORM WATER DRAINAGE WITH CLASS C CONCRETE AT THE RIGHT OF WAY LINE.  PAYMENT FOR PLUGGING SHALL BE INCLUDED IN THE CONTRACT PRICE FOR THE PERTINENT 202 OR 203 ITEM. NOTIFY THE PROJECT ENGINEER PRIOR TO PLUGGING ANY CONDUITS.

FURNISH A CONTINUANCE FOR ALL UNRECORDED TREATED STORM WATER DRAINAGE, SUCH AS ROOF DRAINS, FOOTER DRAINS, OR YARD DRAINS, DISTURBED BY THE WORK. FURNISH EITHER AN OPEN CONTINUANCE OR AN UNOBSTRUCTED CONTINUANCE BY CONNECTING A CONDUIT THROUGH THE CURB OR INTO A DRAINAGE STRUCTURE. THE LOCATION, TYPE, SIZE, AND GRADE OF THE NEEDED CONDUIT TO REPLACE OR EXTEND AN EXISTING DRAIN WILL BE DETERMINED BY THE ENGINEER. ALL SUCH CONTINUANCE REQUIRES

A RIGHT OF WAY USE PERMIT. THE FOLLOWING CONDUIT TYPES MAY BE USED: 707.33, 707.41 NON-PERFORATED, 707.42, 707.43, 707.45, 707.46, 707.47, 707.51, 707.52 SDR35.

FURNISH A CONTINUANCE FOR ALL UNRECORDED ACTIVE SANITARY

SEWER CONNECTIONS SUCH AS SANITARY, WASTEWATER, CURTAIN/

GRADIENT DRAINS, AND FOUNDATION FLOOR DRAINS DISTURBED BY THE WORK.  FURNISH AN UNOBSTRUCTED CONTINUANCE OF THE UNRECORDED ACTIVE SANITARY SEWER CONNECTIONS TO THE SATISFACTION OF THE ENGINEER. ALL SUCH CONTINUANCE REQUIRES A RIGHT OF WAY USE PERMIT. ALL SANITARY AND SANITARY WASTEWATER CONTINUANCE MAY ALSO REQUIRE A NPDES PERMIT FROM THE OHIO ENVIRONMENTAL PROTECTION AGENCY. REPORT ALL CONTINUANCE TO THE LOCAL HEALTH DEPARTMENT. THE FOLLOWING CONDUIT TYPES MAY BE USED: 707.42, 707.43, 707.44, 707.45, 707.46, 707.47, 707.51, 707.52 SDR35, 706.01, 706.02, OR 706.08 WITH JOINTS AS PER 706.11 OR 706.12.

THE FOLLOWING QUANTITIES HAVE BEEN CARRIED TO THE GENERAL SUMMARY,

AS DIRECTED BY THE ENGINEER, FOR THE ABOVE-DESCRIBED PURPOSE:

ITEM 202, PIPE REMOVED, 24” DIAMETER AND UNDER:   20 FT

ITEM 611, 4” CONDUIT, TYPE D:    20 FT