Surveyors Report LOR-252-3.65 PID: 114957

ALIGNMENT:

LOR-252-3.65 Alignment was established using Plans – S.H. (I.C.H) No. 541 Sec. “d” Lorain County R/W Pages 1-3 & LOR-252-3.26 R/W Pages 1-6 Dated 1989. Concrete Monuments MONBOX’s were searched for, located, and measured.

Starting at a PC (MONBOX) Station 178+63.01 on the (South end). Curve 1 established by holding MN023 (PT MONBOX) & MN024 (PC MONBOX) for the forward and back tangents to CLRW08 (PI CALCPT). Record radius was held for the curve (954.93’) LOR-252-3.26 plans.

Curve 1 Record Deflection= 36d 30’ 51” LT

Curve 1 Measured Deflection= 36d 30’ 44” LT

Used record Radius= 954.93’

Forward Tangent 2-line CLRW08 holding MN023 extended to North PI (CLRW09). Tangent 3-line (South end) MN019 (PT Good CMON) held @ 30’, (North end) held record distance of 1003.29’ to PI. Curve 2 created using record radius (819.02’) & Tangent 2 & 3 Lines.

Curve 2 Record Deflection= 07d 38’ 00” RT

Curve 2 Measured Deflection= 07d 43’ 43” RT

Used Record Radius= 819.02’

Tangent 4-line held (South) CLRW10 turning the record deflection 06 degrees 15’ 00” RT & MN018 (Disturbed CMON) Held @ 30’. Record Deflection was used for this Curve (3) due to PC & PT CMONS being Destroyed. Curve 3 created using record radius (1432.69’) and Tangent lines 3 & 4.

Curve 3 Record Deflection= 06d 15’ 00” RT Held Record

Used Record Radius= 1432.69’

STATIONING:

Project Stationed via Existing LOR-252-3.26 Plans. STA. 178+63.01 verified via. Destape.

MONUMENT BOXES TO BE SET:

PT - STA. 193+04.49 Point (CLRW04)

BOUNDARY TAKES:

POC- Point # POC01 (Intersection of Original Lot 37 & Centerline of R/W LOR-252)

Referenced by MN022 ¾” IPIN found 3.825’ @ N89 degrees 35’ 37.1” W

Gregory Tobe P.E.

2-28-24