PID 116919, MIA-41-0132: QUANTITY CALCULATIONS

Calculated by:  Lawton Gerlinger, P.E., Date: 7/23/25

Checked by: Dan Grilliot, P.E., Date: x/x/xx

Revised by:

Roadway, Pavement, Traffic Control DAR-47

- Item 202 Pavement Removed

  - PV-1: 813 SF from Microstation = 813 SF/9 = 91 SY

  - PV-2: 4230 SF from Microstation = 806 SF/9 = 90 SY

    - Total: 91 + 90 = 181 SY

- Item 202 Wearing course removed

  - PV-1: 3500 SF from Microstation = 3500/9 = 389 SY

  - PV-2: 3500 SF from Microstation = 3500/9 = 389 SY

    - Total: 389 + 389 = 778 SY

- Item 204 Subgrade compaction

  - PV-1: 35’ x 25’ = 875/9 = 98 SY

  - PV-2: 35’ x 25’ = 875/9 = 98 SY

    - Total: 196 SY

- Item 209 Reshaping under Guardrail

  - 319’ x 2 sides round up to 800 = 8 Total Stations 2 STA per quad

- Item 301 Asphalt Concrete Base, PG64-22, (449)

  - PV-1: 31.75’ x 25’ x .5 = 397 CF/27 = 15 CY

  - PV-2: 31.75’ x 25’ x .5 = 397 CF/27 = 15 CY

    - Total: 15 + 15 = 30 CY

- Item 304 Aggregate Base

  - PV-1: 32.25’ x 25’ x .5 = 403 CF/27 = 15 CY

  - PV-2: 32.25’ x 25’ x .5 = 403 CF/27 = 15 CY

    - Total: 15 + 15 = 30 CY

- Item 407 Tack Coat (0.055 GAL/SY)

  - PV-1: (25’ x 26’) + (25’ x 26’) + (25’ x 27’) = 1975 SF/9 = 220 SY x 0.085 GAL/SY = 19 GAL

  - PV-2: (25’ x 26’) + (25’ x 26’) + (25’ x 27’) = 1975 SF/9 = 220 SY x 0.085 GAL/SY = 19 GAL

    - Total: 19 + 19 = 38 GAL

- Item 407 Tack Coat (0.085 GAL/SY)

  - PV-1: (125’ x 26’) = 3250 SF/9 = 362 SY x 0.085 GAL/SY = 31 GAL

  - PV-2: (125’ x 26’) = 3250 SF/9 = 362 SY x 0.085 GAL/SY = 31 GAL

    - Total: 31 + 31 = 62 GAL

- Item 441 Asphalt Concrete Surface Course, Type 1, (449), PG70-22M

  - PV-1: 25’ x 31.25’ x 3/12 + 125’ x 26’ x 1.5/12 = 601.56 CF/27 = 23 CY

  - PV-2: 25’ x 31.25’ x 3/12 + 125’ x 26’ x 1.5/12 = 601.56 CF/27 = 23 CY

    - Total = 46 CY

- Item 617 Compacted Aggregate

  - SR-1: 3’ x 125’ x (2/12) + 3’ X 25’ x (8/12) = 113/27 = 5 CY

  - SR-2: 3’ x 100’ x (2/12) + 3’ X 25’ x (8/12) = 100/27 = 4 CY

  - SR-3: 3’ x 125’ x (2/12) + 3’ X 25’ x (8/12) = 113/27 = 5 CY

  - SR-4: 3’ x 125’ x (2/12) + 3’ X 25’ x (8/12) = 113/27 = 5 CY

  - SR-5: 5’ x 75’ x (2/12) = 62.5/27 = 3 CY

    - Total = 22 CY

- Item 626 Barrier Reflector, Type 2 Bi-Directional

  - 100-foot spacing per CMS 626.03. 319 feet per side 319/100 = 3.19 rounds to 4 +1 counting zero. 5 reflectors per side. 10 Total.

- Item 646 Edge Line

  - EL-1: 0.06 Miles. CALC: STA. 968+24.47 – STA. 965+05.47 = 319 FT/5280 FT/MILE = 0.0604 MILE

  - EL-2: 0.06 Miles.

- Item 646 CenterLine

  - CL-1: 0.06 Miles.

- Item 690 Mailbox Support system

  - 1 EACH in SR-5.

Erosion Control

- Item 832 Erosion Control

  - 2000 EACH

- Item 659 Seeding and Mulching

  - XS sheet 1: 139 SY

  - XS sheet 2: 111 SY

Total Seeding and Mulching: 139+111 = 250 SY

- Item 659 Repair Seeding and Mulching

  - 5% of seeding and mulching: 0.05 x 250 = 13 SY

- Item 659 Commercial Fertilizer

  - (1 TON/7410SY permanent) x 250 + (1 TON/11,111 SY interseeding) x 13 = 0.04 TON

- Item 659 Water

  - (0.0027 M. GAL/SY) x 250 + (0.0027 M. GAL/SY) x 13 = 1 M. GAL

Structures: MIA-41-0136

- Item 202 Portions of Structure Removed, APP

  - Lump Sum (LS)

- Item 202 Wearing course removed

  - Existing asphalt on bridge deck: 36.5’ x 19’ = 693.5 SF/9 = 78 SY

- Item 202 Bridge Railing Removed for reuse, APP

  - 20 FT each side (Existing Plans) = 40 FT

- Item 503 Cofferdams and Excavation Bracing

  - Lump Sum (LS)

- Item 503 Unclassified Excavation

  - Lump Sum (LS)

- Item 509 Concrete Reinforcement, replacement of existing concrete reinforcement, as per plan

  - 100 lbs for contingency.

- Item 509 Uncoated Steel Reinforcement

  - Deck Edge rebar: See sheet 16. 898 lbs

- Item 511 Class QC2 Concrete, Bridge Deck

  - 1 Deck edge: 3.29 sf From Microstation, 3.29’ x 19’ x 2 deck edges = 125.02 CF/27 = 5 CY

- Item 516 ½” preformed expansion joint sealer

  - Form between patching and slab edge: (12”/12) x 1’-6” depth x 4 quads = 6 SF

- Item 516 2” Deep Joint Sealer, APP

  - At interface between concrete and asphalt: 36.5’ wide x 1/cos(10) = 37.063 FT x 2 approaches = 74 FT

- Item 517 Railing DBR

  - See sheet 13: 20 FT each side x 2 sides = 40 FT

- Item 518 Steel Drip Strip

  - [19 FT per side + (3 posts x 1.5 FT)] x 2 sides = 47 FT

- Item 519 Patching concrete structure

  - 65 SF Total see sheet 15

- Item 625 structure grounding system

  - 1 EACH

- Item 844 Galvanic Anode Protection, as per plan

  - 20 anodes per deck edge. 40 EACH

- Item 848 Superplasticized dense concrete overlay using hydrodemolition, 2 1/4”

  - 36.5’ x 19’ = 693.5 SF/9 = 78 SY

- Item 848 Surface preparation using hydrodemolition, as per plan

  - 31.5’ x 19’ = 598.5 SF/9 = 67 SY

- Item 848 Superplasticized Dense Concrete overlay (variable thickness), Material only

  - Asphalt on top of existing deck, have to assume a percentage delaminated. 20%

    - 20% per BDM 403-3 table use 40%. 598.5 SF x 2/12” x 40% = 40 CF/27 = 2 CY

- Item 848 Hand chipping

  - 10% of estimated variable thickness area per BDM 403.4.1. 0.4 x 598.50 SF x 0.1 = 23.94 SF/9 = 3 SY

  - Bridge is so small, assume 50% hand chipping = 598.50 SF x .5 = 299.25/9 = 33 SY

- Item 848 Test slab

  - Lump Sum (LS)

Maintenance of Traffic

- Item 614 Detour signing

  - Lump Sum (LS)

Incidentals

- Item 614-Maintaining Traffic (LS)

- Item 624-Mobilization (LS)

No field office confirmed by Michelle 4/20/25 on CD form

END OF CALCULATIONS