UTILITIES

LISTED BELOW ARE ALL UTILITIES LOCATED WITHIN THE PROJECT CONSTRUCTION LIMITS TOGETHER WITH THEIR RESPECTIVE OWNERS:

THERE ARE NO UNDERGROUND UTILITIES SHOWN ON THIS PLAN. THE NATURE OF THE WORK REQUIRED BY THIS PROJECT WILL NOT AFFECT ANY KNOWN UNDERGROUND UTILITIES THAT EXIST UNDER OR ADJACENT TO THE WORK AREA.

CITY OF TROTWOOD

2400 OLIVE ROAD

TROTWOOD, OHIO 45426

(937) 837-1702

JMCCLUSKEY@TROTWOOD.ORG

JOHNNY MCCLUSKEY

FRONTIER COMMUNICATIONS

WILMINGTON, OHIO 45177

(937) 708-9671

ROBIN.LATHAM@FTR.COM

ROB LATHAM

OSP ENGINEERING MANAGER, OHIO

AES OHIO

1900 DRYDEN ROAD

DAYTON, OHIO 45439

(937) 554-9063

WILLIAM.WARD@AES.COM

BILL WARD

MONTGOMERY COUNTY ENVIRONMENTAL SERVICES

1850 SPAULDING ROAD

KETTERING, OHIO 45432

(937) 781-2562

STEWARTK@MCOHIO.ORG

KEN STEWARD

MONTGOMERY COUNTY ENGINEER

451 W. THIRD STREET

DAYTON, OHIO 45422

(937) 225-4904

SHOUPG@MCOHIO.ORG

GARY SHOUP

CHARTER COMMUNICATION

3691 TURNER ROAD

DAYTON, OHIO 45415

(937) 396-8591

EIN.WHATLEY@CHARTER.COM

EIN WHATLEY

CNI FIBER

1720 WILLIPIE STREET

WAPAKONETA, OHIO 45895

(419) 739-3124

SEMANS@CNITEAM.COM

SALES@CNITEAM.COM

SARAH EMANS

PROFILE AND ALIGNMENT

PLACE THE PROPOSED PAVEMENT TO FOLLOW THE ALIGNMENT AND PROFILE OF THE EXISTING PAVEMENT. PLACE THE PROPOSED ASPHALT CONCRETE OVERLAY WITH A UNIFORM THICKNESS OF 1.5 INCHES ON TOP OF EXISTING MILLED AREAS.

ASPHALT CONCRETE SURFACE COURSE SEALING REQUIREMENT

IN ADDITION TO THE GUTTER SEALING REQUIREMENTS SPECIFIED ON SCD BP-3.1 AND IN CMS 401.08, AFTER COMPLETION OF THE SURFACE COURSE, THE CONTRACTOR SHALL SEAL, WITH A CERTIFIED PG BINDER, THE FOLLOWING LOCATIONS:

- ALL CASTINGS INCLUDING BUT NOT LIMITED TO MONUMENTS, MANHOLES, WATER VALVES, CATCH BASINS, CURB INLETS.

- BUTT JOINTS AND FEATHER JOINTS.

- FORWARD JOINT FOR DRIVEWAY ASPHALT AND TRAILING JOINT WHEN BUTTING TO EXISTING ASPHALT DRIVE.

- PERIMETER OF ALL PAVEMENT REPAIRS OR OTHER ASPHALT INLAYS WHEN PAVEMENT REPAIRS/INLAYS ARE NOT OVERLAID WITH AN ASPHALT CONCRETE SURFACE COURSE.

- ALL COLD LONGITUDINAL JOINTS BETWEEN PAVED SHOULDERS AND GUARDRAIL ASPHALT.

- NEW ASPHALT ABUTTS EXISTING CURB OR CURB AND GUTTER

THE MATERIAL USED SHALL BE CERTIFIED 64-22M PG BINDER. THE WIDTH OF THE SEALER SHALL BE 2-3 INCHES.

ANY ADDITIONAL COSTS ASSOCIATED WITH THE WORK IDENTIFIED IN THIS NOTE SHALL BE INCLUDED IN THE APPROPRIATE ASPHALT CONCRETE SURFACE COURSE ITEM OF WORK.

ITEM 254 - PAVEMENT PLANING, ASPHALT CONCRETE

THE PLANING SHALL BE SCHEDULED SO AS TO BE COVERED BY SURFACE COURSE WITHIN TWELVE (12) DAYS. THE COST OF THE ABOVE SHALL BE INCLUDED IN THE UNIT BID FOR THE RESPECTIVE ITEMS. FAILURE TO COMPLY WITH THE ABOVE REQUIREMENTS SHALL RESULT IN THE LIQUIDATED DAMAGES AS PER 108.07 OF THE SPECIFICATIONS.

THE CONTRACTOR SHALL MILL A CONSISTENT 1 1/2" WITHIN THE LIMITS OF THE PROJECT. MILLING AT THESE LOCATIONS IS INCLUDED WITHIN ITEM 254 – PAVEMENT PLANING.

PERMANENT PAVEMENT MARKINGS

THE CONTRACTOR SHALL REFERENCE ALL PAVEMENT MARKINGS INCLUDING AUXILIARY PAVEMENT BEFORE THE START OF THE RESURFACING OPERATION EXCEPT FOR THE EDGE LINE. FOR CENTER LINE MARKINGS, THE CONTRACTOR SHALL INSTALL THE PASSING/NO PASSING ZONE MARKINGS ACCORDING TO THE CURRENT CENTER LINE LOGS AVAILABLE AT THE TIME OF INSTALLATION, THE EDGE LINE SHALL BE REFERENCED OFF THE CENTER LINE IN ACCORDANCE WITH THE LANE WIDTH ON THE PLANS. PAYMENT FOR THIS OPERATION SHALL BE INCLUDED WITH EACH RESPECTIVE PAVEMENT MARKING ITEM.

ITEM 644 - PAVEMENT MARKINGS

ALL PAVEMENT MARKING ON THIS PROJECT SHALL BE REPLACED AT THE SAME LOCATIONS, EXCEPT AS MODIFIED BY THE PLANS. THE CONTRACTOR SHALL BE RESPONSIBLE FOR PROPER LAYOUT AND PRE-MARKING AS PER 641.06 OF THE MATERIAL AND SPECIFICATION HANDBOOK. ALL OTHER PAVEMENT MARKINGS SHALL CONFORM TO ODOT ITEM 644. ALL COSTS INCURRED SHALL BE INCLUDED IN THE UNIT PRICE BID FOR ITEM 644.

ITEM 407 – NON-TRACKING TACK COAT

THE RATE OF APPLICATION OF 407 NON-TRACKING TACK COAT SHALL BE SUBJECT TO ADJUSTMENT, AS DIRECTED BY THE ENGINEER. FOR ESTIMATING PURPOSE ONLY, THE PLAN QUANTITIES INDICATE AN AVERAGE APPLICATION RATE OF: 0.085 GALLONS PER SQUARE YARD OF NON-TRACKING TACK COAT.

TEMPORARY PAVEMENT WEDGE

TEMPORARY PAVEMENT WEDGES SHALL BE PROVIDED AT ALL TIMES WHERE TRAFFIC IS REQUIRED TO TRAVEL FROM OR ONTO PAVEMENT SURFACE OF DIFFERENT ELEVATION. THE MINIMUM SLOPE ON THE TEMPORARY PAVEMENT WEDGE SHALL BE 3:1 ALONG THE LONGITUDINAL JOINTS AND 120:1 AT TRANSVERSE JOINT. THESE WEDGES SHALL BE REMOVED PRIOR TO PLACING THE SPECIFIED PAVEMENT COURSE. PAYMENT FOR ALL WORK, MATERIALS, ETC., ASSOCIATED WITH THIS ITEM SHALL BE PAID UNDER ITEM 614 MAINTAINING TRAFFIC.

DRIVEWAYS

ASPHALT DRIVES SHALL BE TRANSITIONED WITHIN 2-3 FT AND FEATHER JOINTS ARE ACCEPTABLE. ALL CONCRETE DRIVES SHALL BE A BUTT JOINT TYPE. ASSOCIATED COSTS WITH DRIVEWAYS SHALL BE INCLUDED WITH ITEM 441, ASPHALT CONCRETE SURFACE COURSE.

PROTECTION OF THE WOLF CREEK RECREATIONAL TRAIL

A SEGMENT OF THE WOLF CREEK TRAIL CROSSES SHILOH SPRINGS ROAD WITHIN THE PROJECT CORRIDOR.  CONSTRUCTION PERSONNEL SHALL BE ALERT TO CYCLISTS AND ACCOMMODATE CYCLISTS SAFE TRAVEL THROUGH THE INTERSECTION DURING CONSTRUCTION.