# Hydraulic Analysis Report

# Project Data

Project Title: MinervaTrail Bridge

Designer:

Project Date: Monday, November 14, 2024

Project Units:  U.S. Customary Units

Notes:

# Bridge Scour Analysis:B-002-0-23_10 Year_Layer 1_Critical Shear Stress = 0.7360

Notes:

## Scenario: Scour Scenario

### Local Scour at Abutments Summary

#### Left Abutment

Abutment Scour Method:  NCHRP Method

Abutment Scour Depth 5.08 ft

Total Scour at Abutment 5.08 ft

Total Scour Elevation at Abutment 1034.09 ft

### Left Abutment Details

#### Abutment Scour

Computation Type: NCHRP

Input Parameters

##### NCHRP Method

Abutment Type: Vertical-wall abutment

Angle of Embankment to Flow: 90.00 Degrees

Centerline Length of Embankment: 3.75 ft

Projected Length of Embankment: 3.75 ft

Width of Flood Plain: 168.82 ft

Unit Discharge, Upstream in Main Channel (q1): 5.39 cfs

Unit Discharge in the Constricted Area (q2): 5.88 cfs/ft

D50: 1.000049 mm

Upstream Flow Depth: 2.27 ft

User Defined Shear Stress of floodplain

Critical Shear Stress of Floodplain Material: 0.7360 lb/ft^3

Unit Weight of Water: 62.40 lb/ft^3

Manning's n: 0.0300

Flow Depth Prior to Scour: 1.10 ft

##### Result Parameters

q2/q1: 1.09

Average Velocity Upstream: 2.37 ft/s

Critical Velocity above which Bed Materal of Size D and Smaller will be Transported: 1.90 ft/s

Scour Condition: Live Bed

Embankment Length/Floodplain Width Ratio: 0.02

Scour Condition: b (overbank)

Amplification Factor: 2.53

Flow Depth including Contraction Scour: 2.45 ft

Maximum Flow Depth including Abutment Scour: 6.18 ft

Scour Hole Depth from NCHRP Method: 5.08 ft

## Scour Summary Table

### Long Term Degradation

### Contraction Scour

### Local Scour at Piers

### Local Scour at Abutments

| Parameter | Value | Units | Notes |
| --- | --- | --- | --- |
| Left Abutment |  |  |  |
| Abutment Scour Depth | 5.08 | ft | NCHRP Method: Scour Condition B |
| Max Flow Depth including Abutment Scour | 6.18 | ft |  |
| Total Scour at Abutment | 5.08 | ft |  |
| Local Streambed Elevation at Abutment | 1042.33 | ft | prior to any scour |
| Total Scour Elevation at Abutment | 1034.09 | ft |  |

# Bridge Scour Analysis:B-002-0-23_10 Year_layer 2_Critical Shear Stress = 0.0042

Notes:

## Scenario: Scour Scenario

### Local Scour at Abutments Summary

#### Left Abutment

Abutment Scour Method:  NCHRP Method

Abutment Scour Depth 5.08 ft

Total Scour at Abutment 5.08 ft

Total Scour Elevation at Abutment 1034.09 ft

### Left Abutment Details

#### Abutment Scour

Computation Type: NCHRP

Input Parameters

##### NCHRP Method

Abutment Type: Vertical-wall abutment

Angle of Embankment to Flow: 90.00 Degrees

Centerline Length of Embankment: 3.75 ft

Projected Length of Embankment: 3.75 ft

Width of Flood Plain: 168.82 ft

Unit Discharge, Upstream in Main Channel (q1): 5.39 cfs

Unit Discharge in the Constricted Area (q2): 5.88 cfs/ft

D50: 1.000000 mm

Upstream Flow Depth: 2.27 ft

User Defined Shear Stress of floodplain

Critical Shear Stress of Floodplain Material: 0.0042 lb/ft^3

Unit Weight of Water: 62.40 lb/ft^3

Manning's n: 0.0300

Flow Depth Prior to Scour: 1.10 ft

##### Result Parameters

q2/q1: 1.09

Average Velocity Upstream: 2.37 ft/s

Critical Velocity above which Bed Materal of Size D and Smaller will be Transported: 1.90 ft/s

Scour Condition: Live Bed

Embankment Length/Floodplain Width Ratio: 0.02

Scour Condition: b (overbank)

Amplification Factor: 2.53

Flow Depth including Contraction Scour: 2.45 ft

Maximum Flow Depth including Abutment Scour: 6.18 ft

Scour Hole Depth from NCHRP Method: 5.08 ft

## Scour Summary Table

### Long Term Degradation

### Contraction Scour

### Local Scour at Piers

### Local Scour at Abutments

| Parameter | Value | Units | Notes |
| --- | --- | --- | --- |
| Left Abutment |  |  |  |
| Abutment Scour Depth | 5.08 | ft | NCHRP Method: Scour Condition B |
| Max Flow Depth including Abutment Scour | 6.18 | ft |  |
| Total Scour at Abutment | 5.08 | ft |  |
| Local Streambed Elevation at Abutment | 1042.33 | ft | prior to any scour |
| Total Scour Elevation at Abutment | 1034.09 | ft |  |

# Bridge Scour Analysis:B-002-0-23_10 Year_layer 3_D50 = 0.2187

Notes:

## Scenario: Scour Scenario

### Local Scour at Abutments Summary

#### Left Abutment

Abutment Scour Method:  NCHRP Method

Abutment Scour Depth 5.08 ft

Total Scour at Abutment 5.08 ft

Total Scour Elevation at Abutment 1034.09 ft

### Left Abutment Details

#### Abutment Scour

Computation Type: NCHRP

Input Parameters

##### NCHRP Method

Abutment Type: Vertical-wall abutment

Angle of Embankment to Flow: 90.00 Degrees

Centerline Length of Embankment: 3.75 ft

Projected Length of Embankment: 3.75 ft

Width of Flood Plain: 168.82 ft

Unit Discharge, Upstream in Main Channel (q1): 5.39 cfs

Unit Discharge in the Constricted Area (q2): 5.88 cfs/ft

D50: 0.218700 mm

Upstream Flow Depth: 2.27 ft

Flow Depth Prior to Scour: 1.10 ft

##### Result Parameters

q2/q1: 1.09

Average Velocity Upstream: 2.37 ft/s

Critical Velocity above which Bed Materal of Size D and Smaller will be Transported: 1.15 ft/s

Scour Condition: Live Bed

Embankment Length/Floodplain Width Ratio: 0.02

Scour Condition: b (overbank)

Amplification Factor: 2.53

Flow Depth including Contraction Scour: 2.45 ft

Maximum Flow Depth including Abutment Scour: 6.18 ft

Scour Hole Depth from NCHRP Method: 5.08 ft

## Scour Summary Table

### Long Term Degradation

### Contraction Scour

### Local Scour at Piers

### Local Scour at Abutments

| Parameter | Value | Units | Notes |
| --- | --- | --- | --- |
| Left Abutment |  |  |  |
| Abutment Scour Depth | 5.08 | ft | NCHRP Method: Scour Condition B |
| Max Flow Depth including Abutment Scour | 6.18 | ft |  |
| Total Scour at Abutment | 5.08 | ft |  |
| Local Streambed Elevation at Abutment | 1042.33 | ft | prior to any scour |
| Total Scour Elevation at Abutment | 1034.09 | ft |  |

# Bridge Scour Analysis:B-002-0-23_10 Year_layer 4_D50 = 1.2358

Notes:

## Scenario: Scour Scenario

### Local Scour at Abutments Summary

#### Left Abutment

Abutment Scour Method:  NCHRP Method

Abutment Scour Depth 5.08 ft

Total Scour at Abutment 5.08 ft

Total Scour Elevation at Abutment 1034.09 ft

### Left Abutment Details

#### Abutment Scour

Computation Type: NCHRP

Input Parameters

##### NCHRP Method

Abutment Type: Vertical-wall abutment

Angle of Embankment to Flow: 90.00 Degrees

Centerline Length of Embankment: 3.75 ft

Projected Length of Embankment: 3.75 ft

Width of Flood Plain: 168.82 ft

Unit Discharge, Upstream in Main Channel (q1): 5.39 cfs

Unit Discharge in the Constricted Area (q2): 5.88 cfs/ft

D50: 1.235800 mm

Upstream Flow Depth: 2.27 ft

Flow Depth Prior to Scour: 1.10 ft

##### Result Parameters

q2/q1: 1.09

Average Velocity Upstream: 2.37 ft/s

Critical Velocity above which Bed Materal of Size D and Smaller will be Transported: 2.04 ft/s

Scour Condition: Live Bed

Embankment Length/Floodplain Width Ratio: 0.02

Scour Condition: b (overbank)

Amplification Factor: 2.53

Flow Depth including Contraction Scour: 2.45 ft

Maximum Flow Depth including Abutment Scour: 6.18 ft

Scour Hole Depth from NCHRP Method: 5.08 ft

## Scour Summary Table

### Long Term Degradation

### Contraction Scour

### Local Scour at Piers

### Local Scour at Abutments

| Parameter | Value | Units | Notes |
| --- | --- | --- | --- |
| Left Abutment |  |  |  |
| Abutment Scour Depth | 5.08 | ft | NCHRP Method: Scour Condition B |
| Max Flow Depth including Abutment Scour | 6.18 | ft |  |
| Total Scour at Abutment | 5.08 | ft |  |
| Local Streambed Elevation at Abutment | 1042.33 | ft | prior to any scour |
| Total Scour Elevation at Abutment | 1034.09 | ft |  |

# Bridge Scour Analysis:B-001-0-23_10 Year_Layer 1_Critical Shear Stress = 0.1009

Notes:

## Scenario: Scour Scenario

### Local Scour at Abutments Summary

#### Right Abutment

Abutment Scour Method:  NCHRP Method

Abutment Scour Depth 10.62 ft

Total Scour at Abutment 10.62 ft

Total Scour Elevation at Abutment 1023.74 ft

### Right Abutment Details

#### Abutment Scour

Computation Type: NCHRP

Input Parameters

##### NCHRP Method

Abutment Type: Vertical-wall abutment

Angle of Embankment to Flow: 90.00 Degrees

Centerline Length of Embankment: 3.75 ft

Projected Length of Embankment: 3.75 ft

Width of Flood Plain: 138.05 ft

Unit Discharge, Upstream in Main Channel (q1): 2.20 cfs

Unit Discharge in the Constricted Area (q2): 33.84 cfs/ft

D50: 1.000000 mm

Upstream Flow Depth: 1.32 ft

User Defined Shear Stress of floodplain

Critical Shear Stress of Floodplain Material: 0.1009 lb/ft^3

Unit Weight of Water: 62.40 lb/ft^3

Manning's n: 0.0300

Flow Depth Prior to Scour: 4.96 ft

##### Result Parameters

q2/q1: 15.38

Average Velocity Upstream: 1.67 ft/s

Critical Velocity above which Bed Materal of Size D and Smaller will be Transported: 1.74 ft/s

Scour Condition: Clear Water

Embankment Length/Floodplain Width Ratio: 0.03

Scour Condition: b (overbank)

Amplification Factor: 1.13

Flow Depth including Contraction Scour: 13.74 ft

Maximum Flow Depth including Abutment Scour: 15.58 ft

Scour Hole Depth from NCHRP Method: 10.62 ft

## Scour Summary Table

### Long Term Degradation

### Contraction Scour

### Local Scour at Piers

### Local Scour at Abutments

| Parameter | Value | Units | Notes |
| --- | --- | --- | --- |
| Right Abutment |  |  |  |
| Abutment Scour Depth | 10.62 | ft | NCHRP Method: Scour Condition B |
| Max Flow Depth including Abutment Scour | 15.58 | ft |  |
| Total Scour at Abutment | 10.62 | ft |  |
| Local Streambed Elevation at Abutment | 1042.07 | ft | prior to any scour |
| Total Scour Elevation at Abutment | 1023.74 | ft |  |

# Bridge Scour Analysis:B-001-0-23_10 Year_Layer 2_ D50 = 0.303

Notes:

## Scenario: Scour Scenario

### Local Scour at Abutments Summary

#### Right Abutment

Abutment Scour Method:  NCHRP Method

Abutment Scour Depth 10.62 ft

Total Scour at Abutment 10.62 ft

Total Scour Elevation at Abutment 1023.74 ft

### Right Abutment Details

#### Abutment Scour

Computation Type: NCHRP

Input Parameters

##### NCHRP Method

Abutment Type: Vertical-wall abutment

Angle of Embankment to Flow: 90.00 Degrees

Centerline Length of Embankment: 3.75 ft

Projected Length of Embankment: 3.75 ft

Width of Flood Plain: 138.05 ft

Unit Discharge, Upstream in Main Channel (q1): 2.20 cfs

Unit Discharge in the Constricted Area (q2): 33.84 cfs/ft

D50: 0.303000 mm

Upstream Flow Depth: 1.32 ft

Flow Depth Prior to Scour: 4.96 ft

##### Result Parameters

q2/q1: 15.38

Average Velocity Upstream: 1.67 ft/s

Critical Velocity above which Bed Materal of Size D and Smaller will be Transported: 1.17 ft/s

Scour Condition: Live Bed

Embankment Length/Floodplain Width Ratio: 0.03

Scour Condition: b (overbank)

Amplification Factor: 1.13

Flow Depth including Contraction Scour: 13.74 ft

Maximum Flow Depth including Abutment Scour: 15.58 ft

Scour Hole Depth from NCHRP Method: 10.62 ft

## Scour Summary Table

### Long Term Degradation

### Contraction Scour

### Local Scour at Piers

### Local Scour at Abutments

| Parameter | Value | Units | Notes |
| --- | --- | --- | --- |
| Right Abutment |  |  |  |
| Abutment Scour Depth | 10.62 | ft | NCHRP Method: Scour Condition B |
| Max Flow Depth including Abutment Scour | 15.58 | ft |  |
| Total Scour at Abutment | 10.62 | ft |  |
| Local Streambed Elevation at Abutment | 1042.07 | ft | prior to any scour |
| Total Scour Elevation at Abutment | 1023.74 | ft |  |

# Bridge Scour Analysis:B-001-0-23_10 Year_Layer 3_ D50 = 1.6557

Notes:

## Scenario: Scour Scenario

### Local Scour at Abutments Summary

#### Right Abutment

Abutment Scour Method:  NCHRP Method

Abutment Scour Depth 10.62 ft

Total Scour at Abutment 10.62 ft

Total Scour Elevation at Abutment 1023.74 ft

### Right Abutment Details

#### Abutment Scour

Computation Type: NCHRP

Input Parameters

##### NCHRP Method

Abutment Type: Vertical-wall abutment

Angle of Embankment to Flow: 90.00 Degrees

Centerline Length of Embankment: 3.75 ft

Projected Length of Embankment: 3.75 ft

Width of Flood Plain: 138.05 ft

Unit Discharge, Upstream in Main Channel (q1): 2.20 cfs

Unit Discharge in the Constricted Area (q2): 33.84 cfs/ft

D50: 1.655700 mm

Upstream Flow Depth: 1.32 ft

Flow Depth Prior to Scour: 4.96 ft

##### Result Parameters

q2/q1: 15.38

Average Velocity Upstream: 1.67 ft/s

Critical Velocity above which Bed Materal of Size D and Smaller will be Transported: 2.06 ft/s

Scour Condition: Clear Water

Embankment Length/Floodplain Width Ratio: 0.03

Scour Condition: b (overbank)

Amplification Factor: 1.13

Flow Depth including Contraction Scour: 13.74 ft

Maximum Flow Depth including Abutment Scour: 15.58 ft

Scour Hole Depth from NCHRP Method: 10.62 ft

## Scour Summary Table

### Long Term Degradation

### Contraction Scour

### Local Scour at Piers

### Local Scour at Abutments

| Parameter | Value | Units | Notes |
| --- | --- | --- | --- |
| Right Abutment |  |  |  |
| Abutment Scour Depth | 10.62 | ft | NCHRP Method: Scour Condition B |
| Max Flow Depth including Abutment Scour | 15.58 | ft |  |
| Total Scour at Abutment | 10.62 | ft |  |
| Local Streambed Elevation at Abutment | 1042.07 | ft | prior to any scour |
| Total Scour Elevation at Abutment | 1023.74 | ft |  |

# Bridge Scour Analysis:B-001-0-23_10 Year_Layer 4_ D50 = 3.6655

Notes:

## Scenario: Scour Scenario

### Local Scour at Abutments Summary

#### Right Abutment

Abutment Scour Method:  NCHRP Method

Abutment Scour Depth 10.62 ft

Total Scour at Abutment 10.62 ft

Total Scour Elevation at Abutment 1023.74 ft

### Right Abutment Details

#### Abutment Scour

Computation Type: NCHRP

Input Parameters

##### NCHRP Method

Abutment Type: Vertical-wall abutment

Angle of Embankment to Flow: 90.00 Degrees

Centerline Length of Embankment: 3.75 ft

Projected Length of Embankment: 3.75 ft

Width of Flood Plain: 138.05 ft

Unit Discharge, Upstream in Main Channel (q1): 2.20 cfs

Unit Discharge in the Constricted Area (q2): 33.84 cfs/ft

D50: 3.665500 mm

Upstream Flow Depth: 1.32 ft

Flow Depth Prior to Scour: 4.96 ft

##### Result Parameters

q2/q1: 15.38

Average Velocity Upstream: 1.67 ft/s

Critical Velocity above which Bed Materal of Size D and Smaller will be Transported: 2.68 ft/s

Scour Condition: Clear Water

Embankment Length/Floodplain Width Ratio: 0.03

Scour Condition: b (overbank)

Amplification Factor: 1.13

Flow Depth including Contraction Scour: 13.74 ft

Maximum Flow Depth including Abutment Scour: 15.58 ft

Scour Hole Depth from NCHRP Method: 10.62 ft

## Scour Summary Table

### Long Term Degradation

### Contraction Scour

### Local Scour at Piers

### Local Scour at Abutments

| Parameter | Value | Units | Notes |
| --- | --- | --- | --- |
| Right Abutment |  |  |  |
| Abutment Scour Depth | 10.62 | ft | NCHRP Method: Scour Condition B |
| Max Flow Depth including Abutment Scour | 15.58 | ft |  |
| Total Scour at Abutment | 10.62 | ft |  |
| Local Streambed Elevation at Abutment | 1042.07 | ft | prior to any scour |
| Total Scour Elevation at Abutment | 1023.74 | ft |  |

# Bridge Scour Analysis:B-002-0-23_25 Year_Layer 1_Critical Shear Stress = 0.7360

Notes:

## Scenario: Scour Scenario

### Local Scour at Abutments Summary

#### Left Abutment

Abutment Scour Method:  NCHRP Method

Abutment Scour Depth 1.12 ft

Total Scour at Abutment 1.12 ft

Total Scour Elevation at Abutment 1038.01 ft

### Left Abutment Details

#### Abutment Scour

Computation Type: NCHRP

Input Parameters

##### NCHRP Method

Abutment Type: Vertical-wall abutment

Angle of Embankment to Flow: 90.00 Degrees

Centerline Length of Embankment: 3.75 ft

Projected Length of Embankment: 3.75 ft

Width of Flood Plain: 226.45 ft

Unit Discharge, Upstream in Main Channel (q1): 5.52 cfs

Unit Discharge in the Constricted Area (q2): 5.49 cfs/ft

D50: 1.000000 mm

Upstream Flow Depth: 2.49 ft

User Defined Shear Stress of floodplain

Critical Shear Stress of Floodplain Material: 0.7360 lb/ft^3

Unit Weight of Water: 62.40 lb/ft^3

Manning's n: 0.0300

Flow Depth Prior to Scour: 1.36 ft

##### Result Parameters

q2/q1: 0.99

Average Velocity Upstream: 2.22 ft/s

Critical Velocity above which Bed Materal of Size D and Smaller will be Transported: 1.93 ft/s

Scour Condition: Live Bed

Embankment Length/Floodplain Width Ratio: 0.02

Scour Condition: b (overbank)

Amplification Factor: 1.00

Flow Depth including Contraction Scour: 2.48 ft

Maximum Flow Depth including Abutment Scour: 2.48 ft

Scour Hole Depth from NCHRP Method: 1.12 ft

## Scour Summary Table

### Long Term Degradation

### Contraction Scour

### Local Scour at Piers

### Local Scour at Abutments

| Parameter | Value | Units | Notes |
| --- | --- | --- | --- |
| Left Abutment |  |  |  |
| Abutment Scour Depth | 1.12 | ft | NCHRP Method: Scour Condition B |
| Max Flow Depth including Abutment Scour | 2.48 | ft |  |
| Total Scour at Abutment | 1.12 | ft |  |
| Local Streambed Elevation at Abutment | 1040.85 | ft | prior to any scour |
| Total Scour Elevation at Abutment | 1038.01 | ft |  |

# Bridge Scour Analysis:B-001-0-23_25 Year_Layer 1_Critical Shear Stress = 0.1009

Notes:

## Scenario: Scour Scenario

### Local Scour at Abutments Summary

#### Right Abutment

Abutment Scour Method:  NCHRP Method

Abutment Scour Depth 5.88 ft

Total Scour at Abutment 5.88 ft

Total Scour Elevation at Abutment 1032.65 ft

### Right Abutment Details

#### Abutment Scour

Computation Type: NCHRP

Input Parameters

##### NCHRP Method

Abutment Type: Vertical-wall abutment

Angle of Embankment to Flow: 90.00 Degrees

Centerline Length of Embankment: 3.75 ft

Projected Length of Embankment: 3.75 ft

Width of Flood Plain: 146.47 ft

Unit Discharge, Upstream in Main Channel (q1): 4.35 cfs

Unit Discharge in the Constricted Area (q2): 7.50 cfs/ft

D50: 1.000000 mm

Upstream Flow Depth: 2.16 ft

User Defined Shear Stress of floodplain

Critical Shear Stress of Floodplain Material: 0.1009 lb/ft^3

Unit Weight of Water: 62.40 lb/ft^3

Manning's n: 0.0300

Flow Depth Prior to Scour: 1.63 ft

##### Result Parameters

q2/q1: 1.72

Average Velocity Upstream: 2.01 ft/s

Critical Velocity above which Bed Materal of Size D and Smaller will be Transported: 1.89 ft/s

Scour Condition: Live Bed

Embankment Length/Floodplain Width Ratio: 0.03

Scour Condition: b (overbank)

Amplification Factor: 2.18

Flow Depth including Contraction Scour: 3.45 ft

Maximum Flow Depth including Abutment Scour: 7.51 ft

Scour Hole Depth from NCHRP Method: 5.88 ft

## Scour Summary Table

### Long Term Degradation

### Contraction Scour

### Local Scour at Piers

### Local Scour at Abutments

| Parameter | Value | Units | Notes |
| --- | --- | --- | --- |
| Right Abutment |  |  |  |
| Abutment Scour Depth | 5.88 | ft | NCHRP Method: Scour Condition B |
| Max Flow Depth including Abutment Scour | 7.51 | ft |  |
| Total Scour at Abutment | 5.88 | ft |  |
| Local Streambed Elevation at Abutment | 1042.07 | ft | prior to any scour |
| Total Scour Elevation at Abutment | 1032.65 | ft |  |

# Bridge Scour Analysis:B-001-0-23_25 Year_Layer 2_D50 = 0.303

Notes:

## Scenario: Scour Scenario

### Local Scour at Abutments Summary

#### Right Abutment

Abutment Scour Method:  NCHRP Method

Abutment Scour Depth 5.88 ft

Total Scour at Abutment 5.88 ft

Total Scour Elevation at Abutment 1032.65 ft

### Right Abutment Details

#### Abutment Scour

Computation Type: NCHRP

Input Parameters

##### NCHRP Method

Abutment Type: Vertical-wall abutment

Angle of Embankment to Flow: 90.00 Degrees

Centerline Length of Embankment: 3.75 ft

Projected Length of Embankment: 3.75 ft

Width of Flood Plain: 146.47 ft

Unit Discharge, Upstream in Main Channel (q1): 4.35 cfs

Unit Discharge in the Constricted Area (q2): 7.50 cfs/ft

D50: 0.303000 mm

Upstream Flow Depth: 2.16 ft

Flow Depth Prior to Scour: 1.63 ft

##### Result Parameters

q2/q1: 1.72

Average Velocity Upstream: 2.01 ft/s

Critical Velocity above which Bed Materal of Size D and Smaller will be Transported: 1.27 ft/s

Scour Condition: Live Bed

Embankment Length/Floodplain Width Ratio: 0.03

Scour Condition: b (overbank)

Amplification Factor: 2.18

Flow Depth including Contraction Scour: 3.45 ft

Maximum Flow Depth including Abutment Scour: 7.51 ft

Scour Hole Depth from NCHRP Method: 5.88 ft

## Scour Summary Table

### Long Term Degradation

### Contraction Scour

### Local Scour at Piers

### Local Scour at Abutments

| Parameter | Value | Units | Notes |
| --- | --- | --- | --- |
| Right Abutment |  |  |  |
| Abutment Scour Depth | 5.88 | ft | NCHRP Method: Scour Condition B |
| Max Flow Depth including Abutment Scour | 7.51 | ft |  |
| Total Scour at Abutment | 5.88 | ft |  |
| Local Streambed Elevation at Abutment | 1042.07 | ft | prior to any scour |
| Total Scour Elevation at Abutment | 1032.65 | ft |  |

# Bridge Scour Analysis:B-001-0-23_25 Year_Layer 3_D50 = 1.6557

Notes:

## Scenario: Scour Scenario

### Local Scour at Abutments Summary

#### Right Abutment

Abutment Scour Method:  NCHRP Method

Abutment Scour Depth 5.88 ft

Total Scour at Abutment 5.88 ft

Total Scour Elevation at Abutment 1032.65 ft

### Right Abutment Details

#### Abutment Scour

Computation Type: NCHRP

Input Parameters

##### NCHRP Method

Abutment Type: Vertical-wall abutment

Angle of Embankment to Flow: 90.00 Degrees

Centerline Length of Embankment: 3.75 ft

Projected Length of Embankment: 3.75 ft

Width of Flood Plain: 146.47 ft

Unit Discharge, Upstream in Main Channel (q1): 4.35 cfs

Unit Discharge in the Constricted Area (q2): 7.50 cfs/ft

D50: 1.655700 mm

Upstream Flow Depth: 2.16 ft

Flow Depth Prior to Scour: 1.63 ft

##### Result Parameters

q2/q1: 1.72

Average Velocity Upstream: 2.01 ft/s

Critical Velocity above which Bed Materal of Size D and Smaller will be Transported: 2.23 ft/s

Scour Condition: Clear Water

Embankment Length/Floodplain Width Ratio: 0.03

Scour Condition: b (overbank)

Amplification Factor: 2.18

Flow Depth including Contraction Scour: 3.45 ft

Maximum Flow Depth including Abutment Scour: 7.51 ft

Scour Hole Depth from NCHRP Method: 5.88 ft

## Scour Summary Table

### Long Term Degradation

### Contraction Scour

### Local Scour at Piers

### Local Scour at Abutments

| Parameter | Value | Units | Notes |
| --- | --- | --- | --- |
| Right Abutment |  |  |  |
| Abutment Scour Depth | 5.88 | ft | NCHRP Method: Scour Condition B |
| Max Flow Depth including Abutment Scour | 7.51 | ft |  |
| Total Scour at Abutment | 5.88 | ft |  |
| Local Streambed Elevation at Abutment | 1042.07 | ft | prior to any scour |
| Total Scour Elevation at Abutment | 1032.65 | ft |  |