DESIGN CRITERIA TABLE

PROJECT:  STA-SVT 011-BRIDGE 		PID:  118689			Design Speed: 15 mph

| # Type of Data | ## Existing | ## Standard | ## Proposed | ## Comment |
| --- | --- | --- | --- | --- |
| Trail Width | 9’ | 11’ preferred, 8' min. | 9’ | ODOT MDG Table 5-1 The above table allows an 8’”Constrained” width for a LOS D with a peak hour volume of 50 users. This is in line with the traffic volumes of this trail. The existing trail is 9’ wide and the project is only 50’ long. As such, we propose to provide a 9’ trail. If an 11’ trail is provided, transitions will be required, and with a 50’ long project, this results in about 20’ of actual 11’ wide trail. The proposed structure is wide enough to accommodate the 11’ width should the entire trial be widened in the future. Stark Parks is in agreement that a 9’ trail be provided. |
| Graded Shoulder Width | 2.75’ | 3’ preferred, 2’ min. | 2.75’ | MDG 5.3 RM-5.2 |
| Lateral Offset | 0’ min. | 2’ preferred, 1’ min. | 2.25’ min. | MDG 5.3 RM-5.2 |
| SSD (Crest Vertical) | Not in curve | 97’ | 215’ | MDG 3.5.1 MDG Table 5-5 |
| GB% | 20% max. | N/A | 0.46% | This criteria is not listed in MDG |
| Horizontal Curve | R=4080’ | R=41.5’ | R=4080’ | MDG Table 5-4 |
| Cross Slope Transition | 58’/1% | 5’/1% | 58’/1% | MDG 5.3.5 |
| Need for Railing off Structure | Yes, 6’ drop | Yes, 6’ drop | Yes, 6’ drop | MDG 5.3 Railing provided along wingwalls. Once trail is past wingwalls, the slope is 6:1 max. for a width of 5’. Railing is no longer required. |
| Cross Slope | 1.96% max. | 2.00% max. | 1.96% max. | MDG 5.3.5 Matching existing cross slopes |
| Design Speed | N/A | 18-30 mph | 15 mph | MDG Table 5-3 Stark Parks has set a speed limit of 15 mph for their multi-use trails. |

Project Description The proposed project will replace Sippo Valley Trail Bridge #11 with a reinforced concrete three sided flat topped culvert on cast in place reinforced concrete piles. The project includes minimal approach work. The total project length is 50’. The trail is located on a former railbed.