DESIGN CRITERIA TABLE

PROJECT:  STA-SVT 008-BRIDGE 		PID:  118692			Design Speed: 15 mph

| # Type of Data | ## Existing | ## Standard | ## Proposed | ## Comment |
| --- | --- | --- | --- | --- |
| Trail Width | 9’ | 11’ preferred, 8' min. | 9’ | ODOT MDG Table 5-1 The above table allows an 8’”Constrained” width for a LOS D with a peak hour volume of 50 users. This is in line with the traffic volumes of this trail. The existing trail is 9’ wide and the approach work is only ~50’ long on each end. As such, we propose to provide a 9’ trail. The proposed structure is wide enough to accommodate the 11’ width should the entire trial be widened in the future. Stark Parks is in agreement that a 9’ trail be provided. |
| Graded Shoulder Width | Varies 2’-3’ | 3’ preferred, 2’ min. | 2.5’ | MDG 5.3 RM-5.2 |
| Lateral Offset | 0’ min. | 2’ preferred, 1’ min. | 2.5’ min. | MDG 5.3 RM-5.2 |
| SSD (Crest Vertical) | Not in curve | 97’ | Not in curve | MDG 3.5.1 MDG Table 5-5 |
| GB% | 8.68% max. | N/A | 1.83% | This criteria is not listed in MDG |
| Horizontal Curve | R=1775’ | R=41.5’ | R=1775’ | MDG Table 5-4 |
| Cross Slope Transition | 12’/1% | 5’/1% | 28’/1% | MDG 5.3.5 |
| Need for Railing off Structure | Yes, 6’ drop | Yes, 6’ drop | Yes, 6’ drop | MDG 5.3 |
| Cross Slope | 3.59% max. | 2.00% max. | 2.00% max. | MDG 5.3.5 Matching existing |
| Design Speed | N/A | 18-30 mph | 15 mph | MDG Table 5-3 Stark Parks has set a speed limit of 15 mph for their multi-use trails. |

Project Description The proposed project will replace Sippo Valley Trail Bridge #8 with a prefabricated steel truss on concrete abutments founded on cast in place reinforced concrete piles. The project includes minimal approach work. The total project length is 200’. The trail is located on a former railbed.