# Hydraulic Analysis Report

# Project Data

Project Title: Project - SVT #8

Designer:

Project Date: Wednesday, December 4, 2024

Project Units:  U.S. Customary Units

Notes:

# Bridge Scour Analysis:Bridge Scour Analysis - Contraction

Notes:

## Scenario: Q10%

### Contraction Scour Summary

Contraction & Long Term Scour is applied method due to greater scour.

Clear Water Contraction Scour Depth 4.48 ft

### Main Channel Contraction Scour

Computation Type: Clear-Water and Live-Bed Scour

#### Input Parameters

##### Input Parameters for Scour Condition

Average Depth Upstream of Contraction (y1): 4.76 ft

D50 (D50): 0.323393 mm

Average Velocity Upstream (V): 6.08 ft/s

Computed Contraction Scour Condition: Live Bed

##### Input Parameters for Live Bed

Temperature of Water: 60.00 ºF

Slope of Energy Grade Line at Approach Section (S1): 0.002242 ft/ft

Discharge in Contracted Section (Q2): 977.02 cfs

Discharge Upstream that is Transporting Sediment (Q1): 773.69 cfs

Bottom Width in Contracted Section (W2): 68.40 ft

Width Upstream that is Transporting Sediment (W1): 29.95 ft

Depth Prior to Scour in Contracted Section (y0): 3.58 ft

Unit Weight of Water (gamma w): 62.40 lb/ft^3

Unit Weight of Sediment (gamma s): 165.00 lb/ft^3

#### Results

##### Results of Scour Condition

Critical velocity above which bed material of size D and smaller will be transported (Vc): 1.48 ft/s

##### Results of Clear Water Method

Diameter of the smallest nontransportable particle in the bed material (Dm): 0.404241 mm

Average Depth in Contracted Section after Scour (y2): 8.06 ft

Scour Depth (ys): 4.48 ft

##### Results of Live Bed Method

k1 (k1): 0.69

Shear Velocity (V*): 0.59 ft/s

Fall Velocity (V*): 0.14 ft/s

Scour Depth (ys): -0.29 ft

Shear Applied to Bed by Live-Bed Scour (theta 0): 0.0935 lb/ft^2

Shear Required for Movement of D50 Particle (Tau c): 0.0042 lb/ft^2

##### Recommendations

Recommended Scour Condition: Live Bed

Recommended Scour Depth: 0.00 ft

## Scenario: Q4%

### Contraction Scour Summary

Contraction & Long Term Scour is applied method due to greater scour.

Clear Water Contraction Scour Depth 4.48 ft

### Main Channel Contraction Scour

Computation Type: Clear-Water and Live-Bed Scour

#### Input Parameters

##### Input Parameters for Scour Condition

Average Depth Upstream of Contraction (y1): 4.76 ft

D50 (D50): 0.323393 mm

Average Velocity Upstream (V): 6.08 ft/s

Computed Contraction Scour Condition: Live Bed

##### Input Parameters for Live Bed

Temperature of Water: 60.00 ºF

Slope of Energy Grade Line at Approach Section (S1): 0.002242 ft/ft

Discharge in Contracted Section (Q2): 977.02 cfs

Discharge Upstream that is Transporting Sediment (Q1): 773.69 cfs

Bottom Width in Contracted Section (W2): 68.40 ft

Width Upstream that is Transporting Sediment (W1): 29.95 ft

Depth Prior to Scour in Contracted Section (y0): 3.58 ft

Unit Weight of Water (gamma w): 62.40 lb/ft^3

Unit Weight of Sediment (gamma s): 165.00 lb/ft^3

#### Results

##### Results of Scour Condition

Critical velocity above which bed material of size D and smaller will be transported (Vc): 1.48 ft/s

##### Results of Clear Water Method

Diameter of the smallest nontransportable particle in the bed material (Dm): 0.404241 mm

Average Depth in Contracted Section after Scour (y2): 8.06 ft

Scour Depth (ys): 4.48 ft

##### Results of Live Bed Method

k1 (k1): 0.69

Shear Velocity (V*): 0.59 ft/s

Fall Velocity (V*): 0.14 ft/s

Scour Depth (ys): -0.29 ft

Shear Applied to Bed by Live-Bed Scour (theta 0): 0.0935 lb/ft^2

Shear Required for Movement of D50 Particle (Tau c): 0.0042 lb/ft^2

##### Recommendations

Recommended Scour Condition: Live Bed

Recommended Scour Depth: 0.00 ft

## Scour Summary Table

### Long Term Degradation

### Contraction Scour

| Parameter | Q10% | Q4% | Units | Notes |
| --- | --- | --- | --- | --- |
| Selected Contraction Computation Method | Clear-Water and Live-Bed Scour | Clear-Water and Live-Bed Scour |  |  |
| Applied Contraction Scour Depth | 0.00 | 0.62 | ft | Live Bed |
| Clear Water Contraction Scour Depth | 4.48 | 5.41 | ft |  |
| Live Bed Contraction Scour Depth | -0.29 | 0.62 | ft |  |

### Local Scour at Piers

### Local Scour at Abutments

# Bridge Scour Analysis:Bridge Scour Analysis - Abutments SS-2 - SS-5 Left

Notes:

## Scenario: 10%

### Local Scour at Abutments Summary

#### Left Abutment

Abutment Scour Method:  NCHRP Method

Abutment Scour Depth 0.84 ft

Total Scour at Abutment 0.84 ft

### Left Abutment Details

#### Abutment Scour

Computation Type: NCHRP

Input Parameters

##### NCHRP Method

Abutment Type: Vertical-wall abutment

Angle of Embankment to Flow: 90.00 Degrees

Centerline Length of Embankment: 24.00 ft

Projected Length of Embankment: 24.00 ft

Width of Flood Plain: 3.00 ft

Unit Discharge, Upstream in Main Channel (q1): 23.10 cfs

Unit Discharge in the Constricted Area (q2): 18.32 cfs/ft

D50: 0.261750 mm

Upstream Flow Depth: 5.00 ft

Flow Depth Prior to Scour: 4.08 ft

##### Result Parameters

q2/q1: 0.79

Average Velocity Upstream: 4.62 ft/s

Critical Velocity above which Bed Materal of Size D and Smaller will be Transported: 1.39 ft/s

Scour Condition: Live Bed

Embankment Length/Floodplain Width Ratio: 8.00

Scour Condition: a (Main Channel)

Amplification Factor: 1.20

Flow Depth including Contraction Scour: 4.10 ft

Maximum Flow Depth including Abutment Scour: 4.92 ft

Scour Hole Depth from NCHRP Method: 0.84 ft

## Scenario: 4%

### Local Scour at Abutments Summary

#### Left Abutment

Abutment Scour Method:  NCHRP Method

Abutment Scour Depth 0.84 ft

Total Scour at Abutment 0.84 ft

### Left Abutment Details

#### Abutment Scour

Computation Type: NCHRP

Input Parameters

##### NCHRP Method

Abutment Type: Vertical-wall abutment

Angle of Embankment to Flow: 90.00 Degrees

Centerline Length of Embankment: 24.00 ft

Projected Length of Embankment: 24.00 ft

Width of Flood Plain: 3.00 ft

Unit Discharge, Upstream in Main Channel (q1): 23.10 cfs

Unit Discharge in the Constricted Area (q2): 18.32 cfs/ft

D50: 0.261750 mm

Upstream Flow Depth: 5.00 ft

Flow Depth Prior to Scour: 4.08 ft

##### Result Parameters

q2/q1: 0.79

Average Velocity Upstream: 4.62 ft/s

Critical Velocity above which Bed Materal of Size D and Smaller will be Transported: 1.39 ft/s

Scour Condition: Live Bed

Embankment Length/Floodplain Width Ratio: 8.00

Scour Condition: a (Main Channel)

Amplification Factor: 1.20

Flow Depth including Contraction Scour: 4.10 ft

Maximum Flow Depth including Abutment Scour: 4.92 ft

Scour Hole Depth from NCHRP Method: 0.84 ft

## Scour Summary Table

### Long Term Degradation

### Contraction Scour

### Local Scour at Piers

### Local Scour at Abutments

| Parameter | 10% | 4% | Units | Notes |
| --- | --- | --- | --- | --- |
| Left Abutment |  |  |  |  |
| Abutment Scour Depth | -0.51 | 0.84 | ft | NCHRP Method: Scour Condition A (includes LTD) |
| Max Flow Depth including Abutment Scour | 3.07 | 4.92 | ft | Including the long-term scour depth |
| Total Scour at Abutment | 0.00 | 0.84 | ft |  |

# Bridge Scour Analysis:Bridge Scour Analysis - Abutments SS-3 - SS-4 Right

Notes:

## Scenario: 10%

### Local Scour at Abutments Summary

#### Right Abutment

Abutment Scour Method:  NCHRP Method

Abutment Scour Depth 0.84 ft

Total Scour at Abutment 0.84 ft

### Right Abutment Details

#### Abutment Scour

Computation Type: NCHRP

Input Parameters

##### NCHRP Method

Abutment Type: Vertical-wall abutment

Angle of Embankment to Flow: 90.00 Degrees

Centerline Length of Embankment: 42.00 ft

Projected Length of Embankment: 42.00 ft

Width of Flood Plain: 3.00 ft

Unit Discharge, Upstream in Main Channel (q1): 23.10 cfs

Unit Discharge in the Constricted Area (q2): 18.31 cfs/ft

D50: 0.238450 mm

Upstream Flow Depth: 5.00 ft

Flow Depth Prior to Scour: 4.08 ft

##### Result Parameters

q2/q1: 0.79

Average Velocity Upstream: 4.62 ft/s

Critical Velocity above which Bed Materal of Size D and Smaller will be Transported: 1.35 ft/s

Scour Condition: Live Bed

Embankment Length/Floodplain Width Ratio: 14.00

Scour Condition: a (Main Channel)

Amplification Factor: 1.20

Flow Depth including Contraction Scour: 4.10 ft

Maximum Flow Depth including Abutment Scour: 4.92 ft

Scour Hole Depth from NCHRP Method: 0.84 ft

## Scenario: 4%

### Local Scour at Abutments Summary

#### Right Abutment

Abutment Scour Method:  NCHRP Method

Abutment Scour Depth 0.84 ft

Total Scour at Abutment 0.84 ft

### Right Abutment Details

#### Abutment Scour

Computation Type: NCHRP

Input Parameters

##### NCHRP Method

Abutment Type: Vertical-wall abutment

Angle of Embankment to Flow: 90.00 Degrees

Centerline Length of Embankment: 42.00 ft

Projected Length of Embankment: 42.00 ft

Width of Flood Plain: 3.00 ft

Unit Discharge, Upstream in Main Channel (q1): 23.10 cfs

Unit Discharge in the Constricted Area (q2): 18.31 cfs/ft

D50: 0.238450 mm

Upstream Flow Depth: 5.00 ft

Flow Depth Prior to Scour: 4.08 ft

##### Result Parameters

q2/q1: 0.79

Average Velocity Upstream: 4.62 ft/s

Critical Velocity above which Bed Materal of Size D and Smaller will be Transported: 1.35 ft/s

Scour Condition: Live Bed

Embankment Length/Floodplain Width Ratio: 14.00

Scour Condition: a (Main Channel)

Amplification Factor: 1.20

Flow Depth including Contraction Scour: 4.10 ft

Maximum Flow Depth including Abutment Scour: 4.92 ft

Scour Hole Depth from NCHRP Method: 0.84 ft

## Scour Summary Table

### Long Term Degradation

### Contraction Scour

### Local Scour at Piers

### Local Scour at Abutments

| Parameter | 10% | 4% | Units | Notes |
| --- | --- | --- | --- | --- |
| Right Abutment |  |  |  |  |
| Abutment Scour Depth | -0.51 | 0.84 | ft | NCHRP Method: Scour Condition A (includes LTD) |
| Max Flow Depth including Abutment Scour | 3.07 | 4.92 | ft | Including the long-term scour depth |
| Total Scour at Abutment | 0.00 | 0.84 | ft |  |