WAS-26-3.753

Conduit Inventory

| ODOT_DISTRICT | 10 |
| --- | --- |
| COUNTY | WASHINGTON |
| COUNTY_CD | WAS |
| NLFID | SWASSR00026**C |
| CRS | WAS-26-3.753 |
| Route Type | SR |
| Route Number | 00026 |
| Route Suffix |  |
| CTL Begin Number | 3.753 |
| STL Begin Number | 3.753 |
| Latitude DD Begin | 39.430733 |
| Longitude DD Begin | -81.385790 |
| Perp Date | December 30, 2023 |
| Status | A |
| Installation Date | January 1, 1980 |
| Let Plans URL |  |
| Special Designation (Formerly Road ID) | Mainline |
| Conduit File Number | 1885009 |
| Replacing Conduit | N |
| Replaced CFN |  |
| Old Culvert File Number | 840260031 |
| Conduit Owner | S |
| Maintenance Responsibility | S |
| Entry Class | A |
| Features Intersected |  |
| Number of Cells | 1 |
| Broken Back | N |
| Conduit Shape | Circular |
| Conduit Material | Corrugated Metal - Pipe |
| Span (Inches) | 15 |
| Rise (Inches) | 15 |
| Length (Feet) | 96 |
| Metal Gage Thickness | 12 Gauge - 0.109 Inch or 2.77 mm |
| Type of Pipe Protection | Half Bituminous Coated and Paved |
| Slope of Pipe (Percent) | 3.0 |
| Skew (Degrees) | 5 |
| Centerline Span | 15 |
| Inlet End Treatment | None |
| Outlet End Treatment | None |
| Closed System | N |
| Maximum Height of Cover (Feet) | 3 |
| Scour Critical | N |
| Height of Inlet Headwall (Feet) |  |
| Inlet Headwall to Edge Pavement Distance (Feet) |  |
| Height of Outlet Headwall (Feet) |  |
| Outlet Headwall to Edge Pavement Distance (Feet) |  |
| Headwall Critical | N |
| Atmospheric Physical Hazard | N |
| Drainage Area (Acres) |  |
| Drainage Discharge (CFS) |  |
| Abrasion Level | 2 |
| pH |  |
| Conduit Media | \\filestore.dot.state.oh.us\Conduits_Inspection\D10\WASHINGTON\SR\26\1885009 |
| Inventory Comments | needs to be cleaned |
| Modification Type |  |
| Year Modified |  |
| Modification Comments |  |
| PID | 123503 |
| Capital Fiscal Year | 2026 |
| Latest General Apraisal | 5 |
| Latest Inspection Date | March 28, 2024 |
| Last Inspection Year | 2024 |
| Frequency (Years) | 12 |
| Latest Level of Inspection | Inspection from ends of conduit |
| Partial Inspection Indicator |  |
| Inspection Due |  |
| Inventory Photos | View |
| InvPASS_GA_QC |  |
| Level of Inspection | Inspection from ends of conduit |
| Conduit Material Condition | 5 |
| Conduit Alignment | 6 |
| Conduit Shape Rating | 6 |
| Seams or Joints | 6 |
| Slab |  |
| Abutments |  |
| Headwalls |  |
| End Structure |  |
| Channel Alignment | 6 |
| Channel Protection | 6 |
| Waterway Blockage | 4 |
| Scour |  |
| Pavement | 7 |
| Guardrail |  |
| Embankment | 7 |
| General Appraisal | 5 |
| Operational Status | Open - no restriction |
| Inspection Comments |  |
| Inspection Date | March 28, 2024 |
| Recommend Preventive Maintenance |  |
| Recommend Inspection Frequency (Months) |  |
| Inspection Photos | View |
| InpPASS_GA_QC |  |
| P2 General Appraisal |  |
| Image_Attach_Inv |  |
| Image_Attach_Insp |  |
| JURISDICTION | S |
| DIVIDED_HWY | N |
| ACCESS_CONTROL | N |
| URBAN_AREA_CODE | 99999 |
| BASE_TYPE | I |
| FUNCTIONAL_CLASS | 5 |
| HPMS_SAMPLE_ID |  |
| LANES | 2 |
| MAINTENANCE_AUTHORITY |  |
| NHS |  |
| PRIORITY_SYSTEM | G |
| SURFACE_TYPE | G |
| SURFACE_WIDTH | 20 |
| ESAL_TOTAL |  |
| AADT_PASSENGER | 639 |
| AADT_TOTAL | 704 |
| AADT_BC_TRUCK | 65 |
| PAVE_TYPE | FLEX |
| PCR_YEAR |  |
| ROADWAY_WIDTH_NBR | 26 |