WAY-250-25.67

PID  112742

For this project I looked for the concrete R/W monuments called out in our R/W INDEX, and I also looked for monuments at the R/W jogs on the Plan set S.H. 414 SEC. C., MT. EATON & D. (1933). We found a lot of concrete monuments at the R/W jogs.  I held the north concrete R/W monument found at the PI (STA. 343+00). I then held the farthest concrete monument to the west (STA. 316+40) to create the back tangent line. I then held the farthest concrete monument to the east (STA. 369+85.07) to create the forward tangent line.

For stationing I put an even station on the concrete monument closest to the intersection of Zuercher Road using DESKTAPE.  This stationing matched the culvert stationing of the project.

Brett R. Tieben, P.S.

7/27/2023