INTEROFFICE COMMUNICATION

To: David James

From: Nicholas Chaney

Date: February 12, 2026

Subject: Decision Memo: Rehabilitation of SUM-8-7.41 Pedestrian Bridge (PID 121494)

1. Purpose and Background

This memo documents the decision to proceed with a re-decking and rehabilitation of structure SUM-8-7.41 (PID 121494) rather than a full replacement. The bridge, which spans SR 8 and an inactive Metro RTA rail line, has been identified for rehabilitation based on condition-driven needs.

The primary driver for this decision is the need to maximize the utility of limited funding while addressing significant technical and physical hurdles that make a full replacement infeasible.

2. Community Need and Usage

To justify continued investment in this crossing, the District evaluated pedestrian demand via Streetlight data analysis (A. Koenig):

- Daily Volume: Average daily pedestrian volume is 153.

- Conclusion: This volume confirms that the bridge serves a vital connectivity need for the community. Maintaining a safe crossing at this location is essential.

3. Evaluated Alternatives

Option 1: Rehabilitation and Re-decking (Selected)

This option involves replacing the bridge deck while maintaining the existing bridge profile and horizontal footprint.

- Scope: Minimal repair work to the approach walks and a full deck replacement.

- Accessibility Mitigations: Since the existing grade (8.34%) exceeds the maximum profile for a standard ramp without landings, the following safety elements will be added:

  - Double Handrails: Installed where the grade exceeds 5%.

  - High Friction Surface Treatment (HFST): Applied to the deck to improve traction for users navigating the excessive slopes.

- Feasibility: This maintains the crossing at a sustainable cost without triggering extensive right-of-way (R/W) or railroad conflicts.

Option 2: Full Replacement to Standards (Not Recommended)

Building a new bridge to meet current PROWAG/ADA standards (5% to 8.3% grades with landings) was investigated and rejected.

- Railroad Constraints: Metro RTA owns the inactive rail line (Crossing #503578H) beneath the bridge. Standard railroad requirements demand a 23’ vertical clearance. Achieving a compliant grade while maintaining this clearance would require the ramps to extend significantly.

- Physical Impacts: To reach the required clearance with a compliant slope, the ramp length is estimated at 310 feet, likely requiring multi-tiered switchbacks and significant R/W takes.

## 4. Detailed Cost Analysis

- A fiscal comparison was performed to determine the feasibility of full replacement. Standard replacement costs are significantly higher due to the structural complexity of compliant ramps and necessary contingencies.

| Category | Option 1: Rehabilitation | Option 2: Full Replacement |
| --- | --- | --- |
| Base Construction | ~$750,000 - $1,000,000 | $6,000,000 (Structure + Ramps) |
| Contingency (30%) | Included in scope | $1,800,000 |
| Inflation (Mid-2029) | Minimal | $1,000,000 (est. 12.8%) |
| Right-of-Way (R/W) | $0 | $51,000 (Easements) |
| TOTAL ESTIMATED COST | ~$1 Million | $8,851,000 |

- Note: The $1M estimate for rehabilitation does not fully resolve the PROWAG/ADA landing requirements, which are the primary drivers for the $8.8M full-standard estimate.

- Fiscal Conclusion

- Given the pedestrian volume (153/day), an expenditure of $8.8 million is not a prudent use of public funds. The rehabilitation approach (Option 1) provides the necessary structural safety and enhanced user assistance (handrails/HFST) at a fraction of the cost, maximizing the impact of the District’s budget.

5. Railroad Constraints and Feasibility

A critical factor in the decision to rehabilitate rather than replace is the vertical clearance requirement over the Metro RTA rail line (Crossing #503578H).

- Current Status: The line is currently out of service and the Graham Rd. crossing is listed as closed/abandoned on FRA inventory forms.

- Clearance Conflict: To make the bridge ADA/PROWAG compliant, the bridge would need to slope downward over the rail property. However, standard railroad requirements demand a 23’ vertical clearance.

- Risk: While the line appears inactive, unless the line is officially "abandoned" in a legal sense, Metro RTA is unlikely to grant a clearance variance that would preclude future re-activation. Lowering the bridge to meet grade standards is therefore not feasible without a formal abandonment status that has not yet been confirmed.

6. Summary of Decision & Final Scope of Work

The project will proceed as a rehabilitation. We have documented that full compliance with PROWAG profile requirements is "technically infeasible" due to the existing physical constraints of the SR 8 corridor, the Metro RTA vertical clearance requirements, and the disproportionate cost of total reconstruction.

The project scope for PID 121494 will proceed as a rehabilitation:

- Full replacement of the bridge deck.

- Application of High Friction Surface Treatment.

- Installation of double handrails along segments exceeding 5% grade.

- Minor repairs to the approach walkways.

Approved By: Nicholas Chaney

Date: 03/11/2026

GenAI has assisted with this memo.: