Instructions

- The Project Initiation Package is intended to focus on critical issues that can be identified with existing information from secondary sources and/or identified during a site visit.

- Each specialty area of the Project Initiation Package should be completed by individuals who possess sufficient experience to enable them to correctly identify and evaluate issues arising from the field review.

- In the Location/Comments field provide information concerning potential impacts that is brief but gives enough detail to allow an understanding of the issue(s).

- The scope of services document should account for any issues identified in the Project Initiation Package that have the potential to affect scope, schedule, and budget.

- In some instances, resources/subject areas that may need to be consulted for the secondary source review are identified on this form.

Project Initiation Package Deliverables

Provide an expanded Study Area Map identifying project design, utility, right of way and environmental constraints identified through the Project Initiation Package.  Tables, USGS and/or aerial mapping, photographs keyed to available project mapping, the plan to inform and involve the public, and other support material should also be submitted with the Project Initiation Package to illustrate specific problem areas.

General

| ## Date(s) of field review: | ## TBD |
| --- | --- |

| Project Name (County, Route, Section): | POR-14-21.72 | PID: | 124685 |
| --- | --- | --- | --- |
| Date Project Initiation Package Completed: | 2/27/2026 | Prepared By: | Matthew Binion |
| City, Township or Village Name(s): | Atwater, Deerfield, Edinburg, and Palmyra Township | ODOT Project Manager: | Matt Chaney |

Project Description: Safety improvements to the intersections of POR SR 14 at Yale Rd (CR 55) and SR 14 at Alliance Rd (CR 125) in Portage County, Ohio. Safety Study: 20250512-POR-14-21.72-Safety Study

Project Limits/Study Area/General Location: Intersections of POR SR 14 at Yale Rd (CR 55) and SR 14 at Alliance Rd (CR 125) (Google Maps)

| ODOT DISCIPLINE INVOLVEMENT: |  |  |
| --- | --- | --- |
| List name and phone number of individual(s) representing each discipline during the site visit and preparation of the Project Initiation Package. One individual may represent multiple disciplines. |  |  |
| DISCIPLINE | NAME | PHONE NUMBER |
| District Highway Management representative | Mark Griffiths | 330-786-2281 |
| District Planning and Engineering representative | Lauren Phillis Laura Beese | 330-786-4841 330-786-4848 |
| District Environmental Coordinator | Brian Peck | 330-786-4931 |
| District Construction Representative | Joe Schrecengost | 330-603-0393 |
| Geometrics | Kyle Koppes Matt Chaney | 330-786-2253 330-786-4838 |
| Geotechnical | Tom Powel | 330-786-4834 |
| Pavements / Structures | Nicholas Chaney | 330-786-4858 |
| Hydraulics | Mike Palagano Jordan Boehm | 330-786-4851 614-752-0207 |
| TSMO | Aaron Conley | 330-786-4850 |
| Traffic Control | Aaron Conley Michelle Chaney | 330-786-4850 330-786-2267 |
| Utilities | Peter Dinh | 330-786-3132 |
| MOT | Len Blankenship | 330-786-4824 |
| Right of Way / Survey | Tim Ward Brian Honaker | 330-786-4844 330-786-4813 |
| Pedestrian / Bicycle | Matt Chaney | 330-786-4838 |
| EXTERNAL AGENCY INVOLVEMENT: N/A |  |  |
| Indicate external agency involvement during identification of project issues affecting scope development. List the name and phone number of individual(s) representing each agency during the site visit. |  |  |
| AGENCY | NAME | PHONE NUMBER |
| FHWA Engineer*** |  |  |
| Other (LPA, MPO, etc.) |  |  |
| *** The FHWA Engineer should be invited on projects expected to require approval from Federal Highway Administration. |  |  |

| GENERAL EXISTING INFORMATION: Michael Craver |  |  |  |
| --- | --- | --- | --- |
|  | SR 14 | Alliance Rd (CR-125) | Yale Rd (CR-55) |
| Legal Speed: | 55 | 45 | 45 |
| Design Speed: | 60 | 50 | 50 |
| Opening Year ADT (2029): | 6,170 | 1,470 | 330 |
| Design Year ADT (2049): | 6,780 | 1,630 | 360 |
| Trucks (24 Hour B&C): | 6 | 6 | 3 |
| Functional Classification: | Minor Arterial | Local | Local |
| Locale (Rural or Urban): | Rural | Rural | Rural |
| National Highway System (NHS): | No | No | No |

LOCAL PLANNING COORDINATION: Jim Bruner

Briefly describe local planning studies, bike/ped long range plans, aesthetics, etc. that will be considered throughout project development:

Woolpert Safety Study dated May 12,2025.

DISTRICT HIGHWAY MANAGEMENT STAFF CONCERNS: Paul Ensinger / Mark Griffiths / Jeron Hollis

List any comments/requests from the District Highway Management Staff.

Will lighting be at this roundabout? No other comments

| CRASH DATA: Matthew Binion |  |
| --- | --- |
| Has a Safety Study been completed in the project area within past three years | (Yes/No) Yes |
| Is the project area highlighted on the Safety Integrated Project Maps | (Yes/No) No |
| Based on a spatial query (using GCAT or TIMS) of the three most recent years of crash data, briefly summarize crash history including pedestrian and bicycle crashes. Indicate any design features that may be contributing to the observed crash pattern that may be addressed by the project. |  |
| Crash data was obtained from the May 2025 Woolpert Safety Study. For the years 2019-2023 there were a total of 21 crashes, including 13 (62%) injury crashes resulting in one serious injury. The other 8 (38%) crashes were property damage only. There were no fatal crashes during this period There were no crashes in 2019, but the study area has averaged approximately five crashes per year from 2020-2023 Of the 21 crashes, there were 18 (86%) angle, 2 (10%) left turn, and 1 (4%) fixed object crash(es) 16 of the 18 angle crashes occurred at SR 14 and Alliance Rd, including 11 injury crashes. Failure to yield was the highest contributing factor, accounting for 94% of angle crashes at this intersection. The safety improvements at these intersections will reduce crash severity and address the angle crash pattern. For more details, refer to the 20250512-POR-14-21.72-Safety Study |  |

| ENVIRONMENTAL ISSUES: Brian Peck |  |
| --- | --- |
| Make a preliminary determination on whether the following resources are present within the project area. Is it possible that they will be affected by the project. Include the location and any other pertinent information for resources that may be affected. |  |
| Resource/Feature | Location/Comments |
| Parkland, nature preserves and wildlife areas {4(f)/6(f)} | None Present |
| Threatened and Endangered Species and/or habitat | Suitable Wooded Habitat / E. Massasauga Polygon |
| Scenic River | None Present |
| Existing wet areas/existing cattails/wetlands | Present |
| Stream/river/waterway/jurisdictional ditch | Present |
| Historic Resources (buildings, structures, objects) | Possible |
| Historic Bridge(s) | None Present |
| National Historic Landmarks | None Present |
| Archaeological Sites | Unlikely |
| Public Facilities | None Present |
| Cemetery (modern and historic cemeteries) | None Present |
| Farmland | No issues anticipated |
| Watershed Specific (i.e. Darby or Olentangy) NPDES Permit Area | Not Applicable |
| Air Quality non-attainment area or concerns | Possible Qualitative MSAT; no issues anticipated |
| Landfill, Superfund, CERCLIS, RCRA, NPL, or industrial site(s), and/or evidence of hazardous materials | None Present |
| Sensitive environmental justice areas | No issues anticipated |
| Federal Emergency Management Agency (FEMA) floodplains | None Present |
| Lake Erie Coastal Management Area | None Present |
| Sole Source Aquifers | None Present |
| Wellhead Protection Areas | None Present |
| Noise abatement issues | Noise Analysis may be required |
| Coordination with Conservancy Districts | Unlikely |
| Other environmental issues | None Identified |

| RIGHT OF WAY/SURVEY ISSUES: Tim Ward / Brian Honaker Tim Ward / Brian Honaker |  |
| --- | --- |
| Indicate if right of way or survey issues are present or should be considered during project development. Provide additional comments as needed. |  |
| Design Issue | Location/Comments |
| Will there be any work beyond the existing right of way limits? | Yes |
| Will relocation of residences be involved? | No |
| Will relocation of businesses be involved? | No |
| Will the project require modifying the access control to any properties? | Potentially |
| Identify significant right of way encroachments (i.e. large commercial business signs, etc.)? | None seen |
| Will temporary parcels be needed (e.g., for drive work)? | yes |
| Will additional right of way be needed for utility relocations? | potentially |
| Are there any specific property owner concerns? If so, list property owners and concerns. | None at this time. |
| Are work agreements prohibited for any reason? | Work agreements should not be used |
| Are there any other right of way or survey issues? Specify. | None at this time. |

| HYDRAULIC ISSUES: Mike Palagano / Jordan Boehm |  |
| --- | --- |
| Indicate if the following drainage issues are present or should be considered during project development. Side road and service road work should be considered in this assessment. Any available Culvert Inspection reports should be evaluated and attached. Provide additional comments as needed. |  |
| Design Issue | Comments |
| Does the existing drainage system appear to be appropriately sized and functioning properly? Describe deficiencies. | No deficiencies observed in existing system |
| Is there evidence of alignment or flow velocity problems (e.g., scour, bank erosions, silting) at culvert inlets or outlets? | Silt buildup observed at culvert inlets/outlets. This can be addressed with regular culvert channel cleanout |
| Are there sinkholes or other deterioration in the pavement that would indicate separations in the existing pipes? | None observed |
| Is the exposed curb height in existing gutters inadequate to contain flow (include height of proposed resurfacing)? | No curbing present |
| Does the project affect a wetland or waterway (e.g., stream, river, jurisdictional ditch)? | Wetland noted in north corner of intersection. Streams observed on NW and SE legs of SR 14. |
| Will channel relocation be required? | Most likely not, but culvert extension is likely. |
| Will post construction BMPs be required that could impact R/W or utilities? | Yes |
| Are existing underdrain outlets functioning properly? | No evidence of improper UD function. |
| Does the drainage work warrant any special maintenance of traffic considerations? | Nothing beyond what is necessary for RAB construction. |
| Are there any other hydraulic issues? Describe. | Unaware of any other issues. |

| UTILITY ISSUES: Peter Dinh Peter Dinh |  |
| --- | --- |
| Indicate if the following utility issues are present or should be considered during project development. Provide additional comments as needed. |  |
| Design Issue | Location/Comments |
| Do existing utilities need to be relocated? If so, please identify. | Yes, Overhead power and potential underground |
| Would the project benefit from Subsurface Utility Engineering (SUE) Level A? | Yes. Locations TBD |
| Are there existing utilities on an existing structure that need to be relocated? | No |
| Are there any specific utility requirements or concerns? Specify. | Project has large footprint. Utility relocations can be extensive. |
| Are there water or sanitary lines that will be relocated as part of the ODOT contract? | TBD |
| Are there any other utility issues? Specify. | TBD |

| GEOMETRIC DESIGN CONTROLLING CRITERIA (Refer to Section 105 of the LDM, Volume 1): Kyle Koppes / Matt Chaney |  |
| --- | --- |
| Consider design speed, design functional classification, land use, and available traffic data to make a preliminary determination as to the geometric standards for the project and potential for design exceptions. Note exceptions for low volume roadways. |  |
| Design Criteria | Location/Comments |
| Lane Width | Match existing at tie-in locations; Proposed sections in roundabout footprint to follow L&D Vol. 1, Sect. 400 |
| Shoulder Width | Within curb limits, no shoulder; Outside curb limits follow L&D Vol. 1 Sect. 300 |
| Horizontal Curve Radius | Per L&D Vol. 1 Sect. 200 |
| Maximum Grade | Per L&D Vol. 1 Sect. 200 |
| Stopping Sight Distance (Horizontal and Crest Vertical Curves) | Per L&D Vol. 1 Sect. 200 and 400 |
| Superelevation Rate | N/A |
| Vertical Clearance | N/A |
| Pavement Cross Slope | Standard 1.56% on traveled lanes; 2% on circulatory lane and truck apron; also see L&D Vol. 1 |
| Design Loading Structural Capacity | N/A |

| OTHER GEOMETRIC DESIGN ISSUES: Kyle Koppes / Matt Chaney Kyle Koppes / Matt Chaney |  |
| --- | --- |
| Indicate if the following geometric issues are present or should be considered during project development. Consider work on the mainline as well as any side roads or service roads. Provide additional comments as needed. |  |
| #### Design Issues | #### Location/Comments |
| Does the horizontal alignment have an excessive deflection? | No |
| Do the Intersection Angles or Crossroad Alignment meet design standards? | N/A RAB project |
| Is driver comfort an issue due to the vertical curvature or breaks in the grade? | No |
| Does the shoulder width on a structure allow for a minimum width of 4’ from the edge of the traveled way to the face of any barrier? | N/A |
| Has a minimum width of 4’ from the edge of the traveled way to the face of any barrier? | Any proposed replacement or new installation of barrier shall maintain a minimum of 4’ offset. |
| Does intersection sight distance need to be improved? | No |
| List unprotected hazards that appear to be in the clear zone. | Nothing is apparent |
| Should existing access control be revised to improve safety? | No. |
| Are there any drive locations that will require special attention during design (e.g., very steep grades, high volume commercial drives, drives close to bridges or intersections)? | No |
| Do the existing intersection radius returns need to be modified to improve pedestrian crossing safety? | No pedestrian facilities are being installed on this project |
| Do the existing intersection radius returns need to be modified or truck aprons added to accommodate turning movements of large trucks? | No, being a roundabout truck turning movements should be provided and verified. |
| Does grading need to be upgraded? To what criteria (e.g., clear zone, safety, standard)? Consider potential right of way and other impacts when considering grading method. | No |
| Are new or updated curb ramps needed? Refer to the Curb Ramp Measuring Guide | No pedestrian facilities are being installed on this project |
| If constructing a new roadway, will it be a connection between two existing NHS Routes? | N/A |
| If traffic control at an intersection is being changed from stop control to signalization, does the profile of the stop condition road need to be upgraded to accommodate faster traffic? | N/A |
| Are multiple intersection control types being considered? Is an Intersection Control Evaluation (Intersection Control Evaluation (ICE) ¦ Ohio Department of Transportation) applicable? | N/A |
| Are there any other geometric issues? Describe. | The south leg of Alliance Road may need to be tweaked from the study to avoid the proposed road getting too close to the house on the left side. Please investigate alignment and take into account any local regulations for setbacks. Also, pay attention to items such as septic and well locations. However, District 4 has published a roundabout preference guideline. Please review this document and follow our preferences when designing. D04 Roundabout Preferences 20251231.pdf RAB Outside Truck Apron Curbing - 20250311.pdf |

| PAVEMENT ISSUES: Nicholas Chaney Nicholas Chaney |  |
| --- | --- |
| Indicate if the following pavement issues are present or should be considered during project development. Side road and service road work should be considered in this assessment. Provide additional comments as needed. |  |
| Design Issue | Location/Comments |
| Do dynaflect tests indicate the existing pavement is in poor condition? | N/A |
| Are joint repairs needed? | Unlikely. SR 14 was previously paved in 2023/2024 with this segment receiving 1000 SY of partial depth and 500 SY of full depth. |
| Are pressure relief joints needed? | No pressure relief joints at this location. |
| Does curb need to be replaced due to deteriorated condition or lack of curb reveal? | No curb present at this location. |
| Has the site received repeated resurfacings in recent years? | Previous paving projects were 2023 PID 110672 1 ½" mill and fill and 2014 PID 84593 1 ½" mill and fill. |
| Does pavement deterioration appear to be caused by drainage or geotechnical problems? | No. |
| Are there any other pavement issues? Specify. | At this time none are present. Pavement is in very good condition with the next paving project being 8 to 10 years out. |

| GEOTECHNICAL ISSUES: Thomas Powell Tom Powell |  |
| --- | --- |
| Based on the information compiled during this study indicate whether or not the following geotechnical issues are present or should be further considered during project development. Provide additional comments as needed. Refer to Section 302.2 of the ODOT Specifications for Geotechnical Explorations for literature search resources. |  |
| #### Design Issues | #### Location/Comments |
| Is there evidence of soil drainage problems (e.g., wet or pumping subgrade, standing water, the presence of seeps, wetlands, swamps, bogs)? | N/A |
| Will construction be impacted based on the groundwater table? | N/A |
| Is there evidence of any embankment or foundation problems (e.g., differential settlement, sag, foundation failures, slope failures, scours, evidence of channel migrations)? | N/A |
| Is there evidence of any slope instability (soil or rock)? | N/A |
| Is there evidence of unsuitable materials (e.g., presence of debris or man-made fills or waste pits containing these materials, indications from old soil borings)? | N/A |
| Is there evidence of rock strata (e.g., presence of exposed bedrock, rock on the old borings)? | N/A |
| Is there evidence of active, reclaimed or abandoned surface mines? Evidence of quarries? | N/A |
| Is there information pertaining to the existence of underground mines? | N/A |
| Is there Acid Mine Drainage present within the study area? | N/A |
| Are there any other geotechnical issues? Specify. | N/A |

| STRUCTURAL ISSUES: Nicholas Chaney Nicholas Chaney |  |
| --- | --- |
| Indicate if the following structure issues are present or should be considered during project development. Provide additional comments as needed. The Bridge Inspection reports should be evaluated and attached. Provide a separate table for each structure. |  |
| Structure Number(s): |  |
| Design Issue | Location/Comments |
| Is it possible for the structure to be replaced with a prefabricated box culvert or 3-sided box? | N/A |
| Is the deck delaminated? Specify. | N/A |
| Is non-destructive testing needed to determine the Amount of delamination? | N/A |
| Are there areas to be patched/repaired on the deck? | N/A |
| Is the bridge a poor candidate for an overlay? Specify type of overlay if known. | N/A |
| Does the bridge rail violate current standards? | N/A |
| Is fatigue analysis required? | N/A |
| Should all fatigue prone details be retrofitted or replaced? Specify. | N/A |
| Is there any evidence of substructure movement (e.g., settlement, rotation)? | N/A |
| Is elimination of the deck joint possible? What modifications are necessary? | N/A |
| Is it possible for the hinges to be removed to make the members continuous? | N/A |
| Is there any evidence that the bridge does not meet hydraulic capacity? | N/A |
| Are there existing sidewalks on or adjacent to the bridge? | N/A |
| Is Vandal Protection Fencing required in accordance with the BDM? | N/A |
| Will the structure work require any special maintenance of traffic (e.g., closing of roadway for erection of beams, maintenance of waterway traffic, location of cut line, etc.)? Specify. | N/A |
| Does the bridge need to accommodate future roadway lanes, bicycle lanes, a shared use path, shoulder, or railroad tracks? | N/A |
| Will temporary shoring be required next to the railroad? | N/A |
| Describe any issues with the bridge deck (curb, sidewalk, railing, surface, median, drainage, expansion joints, etc.). | N/A |
| Describe any issues with the bridge superstructure (alignment, beams/girders/slab, bearing devices, etc.). | N/A |
| Describe any issues with the bridge substructure (abutments, piers, backwalls, wingwalls, scour, etc.). | N/A |
| Describe any issues with the channel (i.e. alignment, erosion, etc.) | N/A |
| Describe any issues with the bridge approaches (i.e. pavement, guardrail, etc.) | N/A |
| Are there any other structure related issues? Specify. | N/A |

| TSMO CONSIDERATIONS: Aaron Conley Aaron A Conley |  |
| --- | --- |
| Briefly describe the opportunities for managing congestion or traffic issues using TSMO strategies or improvements. Consider opportunities to upgrade or install systems management and operations infrastructure: TSMO infrastructure includes communications equipment, travel time signs, signals, changeable message signs, traffic cameras, traffic signal systems, other remote field devices and data collection equipment, conduit and any supporting fiber optics. TOAST is the Traffic Operations Assessment System Tool. For additional TSMO information see http://www.dot.state.oh.us/Divisions/Operations/Traffic/miscellaneous/Pages/TSMO.aspx |  |
| Design Issue | Location/Comments |
| Does the project area contain a Hot Spot identified in TOAST? If so, what is the TOAST ranking? | ## SPORSR00014**C_17.559_25.342_F (Score: 67% / SW Overall Rank: 4052) ## SPORSR00014**C_17.559_25.342_R (Score: 72.25% / SW Overall Rank: 5935) |
| Does the project area have an operations master plan (or has this site been discussed with the District TSMO Coordinator)? | No |
| Would operations benefit from TMC coverage of the project area? (RWIS, travel time boards, cameras, communications) | No |
| Are there opportunities for initiating or upgrading TSMO infrastructure? | No |
| Does this project support any TSMO strategies such as (Smartlane, VSL, Coordinated traffic signals, etc.) | No |
| Does this project require multi-jurisdictional coordination, agreements, funding, etc.? | No |
| What existing TSMO infrastructure is in place? Will it need to be moved or maintained in place? | None |
| Are there any local TSMO infrastructure recommendations in the project area? (ex. Include emergency or transit traffic signal pre-emption, dynamic message signs or signal coordination) | No |
| What MPO ITS architecture is already in place or planned? Consult the MPO ITS architecture plan, if applicable. | None |
| Categories of potential ITS for this study area/project include: Exempt, Low, or High risk? Ref: TEM, 1-pager for CFR 940. | Exempt |
| Could this project expand an existing device or communications system? | No |
| What type of device communications and equipment exists? | None |
| Should this location have communications added or upgraded? | No |
| Will additional conduit be necessary for future infrastructure/communications? (ex. in barrier wall) | No |
| Will existing device power or communications drops be disrupted? | No |
| Does this project require a new traffic signal timing plan? | No |
| Are the current traffic signal(s) being upgraded to a system? | N/A |
| Are there alternative routes available/identified for incident management? | SR 225 or SR 183 |
| Is this a Traffic Incident Management Note eligible project? | No |
| OTHER TSMO Considerations: None |  |

| TRAFFIC CONTROL ISSUES: Aaron Conley Michelle Chaney |  |
| --- | --- |
| Indicate if the following traffic control (signals, signing, pavement markings, etc.) issues are present or should be considered during project development. Provide additional comments as needed. |  |
| Design Issue | Comments |
| Are there any obvious deviations from requirements of the Ohio Manual of Uniform Traffic Control Devices (OMUTCD)? | No |
| Will coordination with Ohio Rail Development Commission (ORDC) be required (i.e. at-grade railroad crossings located within 400' of an intersection within the project area)? | No |
| Will pavement widening affect pole locations? | Likely due to truck aprons |
| Will resurfacing affect signal height? | No |
| Does it appear that any traffic control items will fall outside the existing right of way limits (e.g., large signs, strain poles)? | No |
| Are there any crashes that can be related to existing signal deficiencies (e.g., timing, lack of protected turn phase)? | No |
| Do pedestrian signals and push buttons need to be installed or upgraded? | No |
| Do turn lane lengths appear to have sufficient storage capacity? | N/A |
| Does the controller need to be upgraded? | No |
| Do proprietary materials need to be specified? | No |
| Should signs or signal installations be supplemented with lighting? | No |
| Are any Tourist Oriented Directional Signs (TODS) or LOGO signs present? | Yes, TODS on SR 14 EB |
| Are there any other traffic control issues? Specify. | No |

| MAINTENANCE OF TRAFFIC ISSUES: Leonard Blankenship Len Blankenship |  |
| --- | --- |
| Indicate if the following maintenance of traffic issues are present or should be considered during project development. Provide additional comments as needed. |  |
| Design Issue | Location/Comments |
| Are there bridge load limits within the work limits or in the nearby area that would limit the available signed official detour or unsigned local alternate routes? | The official detour route will not be affected by bridge weight limits. There are weight limits on local roads that may see increased traffic. District will permit a total intersection closure (up to 120 days) for roundabout construction. A lesser duration closure of SR-14 would be desirable. |
| Is the project located on the National Truck Network? | POR-14 is on the National Truck Network |
| Are there overhead bridges with existing vertical clearance issues or that may become vertical clearance issues (e.g. shifting traffic to the shoulder, adding pavement without milling first, etc.) | N/A |
| Are there pinch points within the work area that that would prevent the installation of temporary pavement for maintaining the existing number of lanes? If yes, identify the location and type of width restraints. (e.g., median wall, at grade bridge, overhead bridge piers, trees, historic markers, etc.) | Temporary pavement placement would be limited by the proposed R/W width and final location of the proposed roundabout. |
| Are there visible signs of pavement condition deterioration in the driving lanes? On the shoulders? If yes, identify location and estimated degree of deterioration and if further testing is needed. | N/A |
| Are there nearby schools that may be adversely impacted by the proposed work? If yes, identify names, location and school districts. | Yes. Waterloo Local School District: 1464 Industry Road, Atwater 44201 (330) 947-2664 Southeast Local Schools: 8245 Tallmadge Rd, Ravenna 44266 (330) 654-5841 |
| Are there nearby emergency services (e.g., hospital, fire, police, EMS, etc.) that may be adversely impacted by the proposed work? If yes, identify locations and names. | Emergency services are provided by County and Township agencies. Expected to affect Portage County Sheriff’s Department. Edinburg Twp., Palmyra Twp., Deerfield Twp., and Atwater Twp. |
| Are there significant traffic generators nearby that may be adversely impacted by the proposed work? (e.g., industries, factories, sports arenas, etc.) | No |
| What is the width of the existing pavement? Will temporary pavement be needed to maintain the existing number of travel lanes? | The width of SR-14 existing pavement is 24’ plus shoulders. It is expected that traffic will be maintained via detour. Temporary pavement may be used if options other than a full intersection closure are selected. |
| What geometric features exist within the work area and within the area of influence of the work area that may impact sight distances and/or flow of traffic? (e.g., horizontal/vertical curves, blind driveways, intersections, entrance/exit ramps, railroad crossings, etc.) | N/A |
| Are there sidewalks or paths within or leading to/from the work area that need to be closed? | No |
| If sidewalk/path needs to be closed, can users be detoured on the existing sidewalk system or will a temporary pedestrian and/or bicycle pathway need to be included in the plan? | N/A |
| Are transit stops present within the work area? | No |
| Are there culverts within the work area that may need to be lengthened to accommodate temporary widening? If so, identify locations and culvert numbers. | A culvert is present on SR-14 at SLM 21.671 (north side of the intersection). Widening for MOT efforts is not anticipated. |
| Are there any known existing drainage issues within the work limits? If yes, special attention needs to be given to ensuring temporary drainage can be accomplished. | No. |
| Will personal and/or business driveways be adversely impacted or need to be closed for any amount of time? | Driveway impacts are dependent on the final roundabout configuration. Access to affected properties will be maintained during construction. |
| Is the project located in or nearby an area of regional significance with a potential to cause controversy or negative public feedback or political scrutiny? | No. |
| Is there enough width to provide safe construction access? If no, what other means of access can be provided? | Yes. |
| Is there potential for the need to require right-of-way acquisition? | R/W acquisition will not be driven by MOT efforts. |
| Is there room in the median for the construction of crossover pavement within the project limits and beyond the project limits on either end? If yes, identify potential locations for crossover locations. | N/A |
| Are short duration road closures going to be required? (e.g., bridge demo, steel erection, overhead utility installation/removal, etc.). If yes, is there an opportunity for diversion of the traffic to other routes or to the ramps on a diamond interchange? Identify the potential diversion routes. | N/A |
| Will there be a need for temporary structures (full or partial) in order to maintain the existing number of lanes? | No. |
| Is there power available within or nearby the project location for temporary lighting and/or temporary signals? | Yes |
| Will there be a need for additional signal heads (drives and/or side roads) or temporary signal timing/coordination? | No |
| Are there any Traffic Incident Management features, such as hydrants, pull-offs, turn-arounds, etc.? | No |
| Are there issues that may limit the construction timeframe? (e.g., sporting or other significant regional events, work in streams, suitable wooded habitat, school, etc.). If yes, list them. | Ideally, the detour/closure duration will be scheduled to minimize the impact on local schools. |
| Would this project potentially benefit from the application of innovative contracting method (e.g., A+B to open bridge to traffic before school starts, etc.)? If yes, which method? | A window contract would be considered. District will allow up to a 120-day total closure of the intersection. Determine if the closure may be reduced to 90 days. Evaluate the completion of offline improvements to minimize the closure duration of SR-14 while Yale Road and Alliance Road are closed. |
| Will there be a need to restrict existing movements during construction? (e.g., no left turns, etc.) | Traffic will most likely be detoured during construction. |
| Is there an opportunity (or potential need) to implement any work zone ITS components? (e.g., work zone egress warning, queue detection and warning, CCTV, DDMS, etc.) | No. |
| How big of an impact will the project have on queue lengths and congestion? If significant, a MOT Policy Exception Request may be required per Traffic Management in Work Zones Policy (21-008(P)) and Standard Procedure (123-001(SP)). | N/A |
| Does this project require an MOTAA? All Path 4 & 5 projects along with Path 3 projects on Interstate/Interstate look-alikes need to have a Maintenance of Traffic Alternatives Analysis Completed. Refer to TEM Section 630-5 | No. |

| CONSTRUCTION ISSUES: Joe Schrecengost |  |
| --- | --- |
| Indicate if the following issues are present or should be considered during project development. Provide additional comments as needed. |  |
| Issue | Location/Comments |
| Will any of the construction activity take place over, under, or near railroad property? | No |
| Could material with long lead times for delivery have an impact on the construction schedule and/or project completion (e.g., strain poles, large box culverts, steel beams, etc.)? | Not Likely, Light Poles could have a lead time but the standard poles should not be an issue. |
| Are there any concerns related to existing or proposed lighting (e.g., light trespass, river navigation, airway clearance)? | No |
| Compare the Begin/End construction dates with the Scope of Work. Is the construction schedule reasonable? | Currently only showing 1 month of construction. This project will likely be completed under a detour with a duration between 90-120 days. The schedule will need to reflect this. |
| Examine the existing pavement condition and repair history. Calculate potential pavement repair quantities. | None needed if full depth construction for roundabout. |
| Note manhole lid elevations versus proposed paving thickness. Will manhole lids or valve boxes need adjusted after paving? | N/A |
| Is there a need for Echelon Paving? | No |
| Examine the rideability of the approach slab to the roadway/bridge joint. | N/A |
| Will the project have impacts to nearby residents/businesses? Will site access occur down steep side slopes or through properties adjacent to project site? | There are adjacent residents which will be affected. No businesses in the immediate vicinity of the project. No access issues. |
| Examine existing guardrail condition, height and length of need. What is the condition of the slopes behind guardrail? Will additional grading or fill be required for guardrail replacement? | N/A |
| Is more space or room needed for construction? Is Temporary or Permanent R/W required for utility relocations, construction of structures, drainage ditches, etc.? | R/W seems large enough. May need adjusted based on final design. |
| Is there enough clearance to overhead utility lines for cranes and concrete pump trucks? | N/A |
| Will there be instream work? | Not Likely |
| Will Temporary shoring/sheeting, cofferdams or work pads be required to complete the proposed work? Anticipated Permitting (see Agency Coordination/Permit Issues section above) | No |
| Will the road need to be detoured to complete construction? What are the possible detour routes? | Yes. Detour route would be either 224 to SR183 or 224 to SR225 to I76. |
| Where are the potential staging areas for the contractor? | Within road closure or ODOT facility nearby if allowed by HMA. |

| PEDESTRIAN AND BICYCLE ISSUES: Matt Chaney |  |
| --- | --- |
| Indicate if the following pedestrian and bicycle facilities are present or should be considered for implementation during project development. - Pedestrian facilities: sidewalks, shared use paths, enhanced crossings, signs/signals, and lighting. - Bicycle facilities: bike lanes, improved shoulders, shared use paths, crossing treatments, signs/signals, and lighting. Provide additional comments as needed. For additional bicycle and pedestrian data, see the TIMS Active Transportation Map Viewer: https://gis.dot.state.oh.us/tims/Map/ActiveTransportation and discuss with the District Bike & Ped Contact. |  |
| Issue | Location/Comments |
| Are there visible signs of deterioration on sidewalks or missing sidewalks? | No |
| Is there a minimum 4’ clearance along sidewalks? (i.e. poles that obstruct the sidewalk) | N/A |
| Are there visible sign of deterioration in bike lanes/shoulders or missing bike facilities? | No |
| Do crossings for bicyclists and/or pedestrians need to be improved or installed? | No |
| Is on-street parking set back 20 feet from the crosswalk (both marked and unmarked) at an intersection or set back 30 feet of the approach to any flashing beacon, stop sign or traffic control device? (See ORC 4511.68) | N/A |
| Is there evidence of the need for a midblock crossing? (i.e. pedestrian crashes, signalized intersection spacing exceeds 600 ft., presence of midblock transit stops or path, pedestrian generators and destinations). Refer to FHWA Guide for Improving Pedestrian Safety at Uncontrolled Intersections | No |
| Does the project area have an active transportation plan in place (or other multimodal plan such as a bicycle, pedestrian, school travel plan, or metropolitan transportation plan). Contact pertinent local public agencies for more information. | No |
| Is there existing bicycle or pedestrian usage along this corridor? (For statewide volume data visit ODOT’s Non-Motorized Database System.) Visible indicators of usage include counts, worn paths, transit stops, etc. | No |
| Is the project located on a designated or proposed bike route (local, regional, state or US)? | No |
| What is the Level of Traffic Stress (1-4)? (LTS 1 and 2 are considered comfortable for the mainstream adult population.) (See Level of Traffic Stress calculation tool. This data is pre-calculated for the State & US Bike Route System.) | N/A |
| Does the project area have high Active Transportation Demand and high Active Transportation Need (Scores of 3 or 4)? (Use the Identify Features tool to select project area and view scores for Demand_ Mapping and Need_Mapping. scores.) | No |
| What are the proposed bicycle lane widths? | N/A |
| What are the proposed sidewalk and shared use path widths (and buffer width)? | N/A |
| If bike/ped accommodations require additional ROW not planned for the project, can a future project provide this? | N/A |

| AGENCY COORDINATION/PERMIT ISSUES: Brian Peck |  |
| --- | --- |
| Indicate if the following permit issues are present or should be considered during project development. Provide additional comments as needed. |  |
| Issue | Location/Comments |
| Will an Individual US Army Corps of Engineers/ Environmental Protection Agency 404/401 permit be required? | Likely USACE Pre-Construction Notification (NWP Linear 14) |
| Will a Section 408 Permission be required for work within an USACE Civil Works (dams, levees, locks, navigation channel, etc.)? Refer to the National Levee Database (army.mil); National Inventory of Dams (army.mil); Louisville District (arcgis.com) Not all projects are found within these directories. Consult with OES during planning to discuss Section 408 coordination. (Note, Section 9 or Section 10 permit will most likely trigger Section 408 coordination.) | No |
| Will a Coast Guard (Section 9) permit be required? | No |
| Is review by a local public agency or project sponsor required? Specify. | Portage County Engineer’s Office |
| Is State Historic Preservation Office (SHPO) coordination for work involving historic bridges or historic properties required? | Possible |
| Is coordination with ODNR for work involving State Scenic Rivers, State Wildlife Areas or State Recreational Areas required? | No |
| Is coordination with any other agency required? | None Identified |

| SCOPE, SCHEDULE AND BUDGET CONSIDERATIONS: Jim Bruner |  |
| --- | --- |
| Based on the responses to the above items, do any of the following need to be modified? |  |
| Issue | Comments |
| Conceptual scope | No |
| Work limits | No |
| Probable environmental document type | C2 |
| Project Path classification | Path 2 |
| Schedule | No |
| Budget | No, has $4.2m in Safety funds for Construction in FY 2030 |