Bridge #4 (GA = 6)

Approach:
Both the north and southbound rails are completely overgrown with vegetation.
The roadway underneath the bridge has major map cracking with several depressed areas throughout the S bound lane in addition to moderate map cracking with partially patched areas throughout the N bound lane.
The sidewalks underneath the bridge has major deterioration along the N bound lane with minor deterioration and several diagonal cracks along the S bound lane.

Deck:
Has a widespread amount of weathered and rotting boards, with several broken and missing, along the N-S outside channels as well as a widespread amount of weathered and rotting railroad ties, with significantly large checks and splits, throughout.
 	*Inspector recommends deck replacement and/or modification.

Superstructure:
Girders #1+2 – Have a widespread amount of surface rust with paint peeling and minor section loss throughout. Has several bent and deflected stiffeners along the inbound and outbound sides of the webs.
Stringers – Has 2 stringers per bay, for 6 bays, with a widespread amount of surface rust with paint peeling and minor section loss throughout.
Floorbeams #1-5 – Have plated flanges with a widespread amount of surface rust with paint peeling and minor section loss throughout. Has several isolated areas of moderate section loss at the angle iron to the bottom flanges.
Lower Lateral Bracing #1-6 – Has a widespread amount of surface rust, paint peeling, and minor section loss throughout with visible sign of pack rust between the connection plates. Has bent and deflected braces in bays #2-3-4-5-6. Has bent and deflected connection plates at the N side of floorbeams #3-4-5 and the S side of floorbeams #1-4.
Bearings – Has paint peeling with surface rust and minor section loss.
 	*Inspector recommends painting the superstructure.

Substructure:
E. wall, forward abutment, has 5 rows of cracked and partially broken stones – missing up to 6” of the faces. Has a concrete cap with hairline map cracks, efflorescence, and isolated areas of spall throughout.
W. wall, rear abutment, has 5 rows of cracked and partially broken stones – missing up
to 7” of the faces. Has a concrete cap with hairline map cracks, efflorescence, and isolated areas of spall throughout. The NW wing has an area of significantly deteriorating stones, leaving joint voids up to 12” deep.
 	*Inspector recommends regrouting the joints and encasing the NW wing.