Prefinal

Inspection of BUT 129 2561C (0901816)

Items found:

- Rear bearing at beam 9 has some lateral movement.

- HL vertical cracks in rear abutment, most sealed.

- Unfilled dowel hole from form work on rear face of pier above column 6.

- Attachments on bottom flanges of beams 1 and 9, west of pier fwd. abutment.

- Trash, construction material between parapet walls along pier.

- Few HL vertical cracks. One under beam 5 with efflorescence, staining.

- Beam 6 in span 2, close to splice plate has an area with odd color to weathering stee.

- Areas of right-of-way fence not completed at the south and north ends of the east fence.

- Sealed longitudinal cracks in slabs, also some areas of scaling.

- Striping on wear surfaces is light, worn off throughout.

- Sealed and few unsealed transverse cracks in wearing surfaces.

- Honeycomb area on the bottom of the pier cap.

- Debris in expansion joints.

- Asphalt on concrete wearing surface.

- Some fence anchor bolts are cut and not touched up with a zinc paint.

Photos can be viewed at; J:\structures\bridges\23 photos\BUT\SR129\2561C

Stephanie Lentz